Vostro 15 3530 
Owner's Manual
Regulatory Model: P112F
Regulatory Type: P112F010/P112F011
March 2024
Rev. A03

Notes, cautions, and warnings
NOTE:  A NOTE indicates important information that helps you make better use of your product.
CAUTION:  A CAUTION indicates either potential damage to hardware or loss of data and tells you how to avoid 
the problem.
WARNING:  A WARNING indicates a potential for property damage, personal injury, or death.
© 2023-2024  Dell Inc. or its subsidiaries. All rights reserved. Dell Technologies, Dell, and other trademarks are trademarks of Dell Inc. or its 
subsidiaries. Other trademarks may be trademarks of their respective owners.

Chapter 1: Views of Vostro 15 3530 ............................................................................................... 7
Right ........................................................................................................................................................................................ 7
Left.......................................................................................................................................................................................... 7
Top.......................................................................................................................................................................................... 9
Front ...................................................................................................................................................................................... 10
Bottom .................................................................................................................................................................................. 12
Service Tag .......................................................................................................................................................................... 12
Battery charge and status light ..................................................................................................................................... 13
Chapter 2: Set up your Vostro 15 3530 ......................................................................................... 14
Chapter 3: Specifications of Vostro 15 3530 ................................................................................ 16
Dimensions and weight ..................................................................................................................................................... 16
Processor ............................................................................................................................................................................. 16
Chipset .................................................................................................................................................................................. 17
Operating system ............................................................................................................................................................... 17
Memory ................................................................................................................................................................................. 18
External ports ...................................................................................................................................................................... 18
Internal slots ........................................................................................................................................................................ 19
Ethernet ............................................................................................................................................................................... 19
Wireless module .................................................................................................................................................................. 19
Audio ..................................................................................................................................................................................... 20
Storage ................................................................................................................................................................................. 21
Media-card reader ............................................................................................................................................................. 21
Keyboard .............................................................................................................................................................................. 21
Keyboard shortcuts of Vostro 15 3530 ........................................................................................................................ 22
Camera ................................................................................................................................................................................. 23
Touchpad ............................................................................................................................................................................. 23
Power adapter .................................................................................................................................................................... 24
Battery ................................................................................................................................................................................. 25
Display .................................................................................................................................................................................. 26
Fingerprint reader (optional) .......................................................................................................................................... 27
Sensor ................................................................................................................................................................................. 27
GPU—Integrated .............................................................................................................................................................. 27
GPU—Discrete .................................................................................................................................................................. 28
Hardware security ............................................................................................................................................................. 28
Operating and storage environment ............................................................................................................................. 28
Dell support policy ............................................................................................................................................................. 28
ComfortView ...................................................................................................................................................................... 29
Chapter 4: Working inside your computer .................................................................................... 30
Safety instructions ............................................................................................................................................................ 30
Before working inside your computer ..................................................................................................................... 30
Safety precautions ....................................................................................................................................................... 31Contents
Contents 3

Electrostatic discharge—ESD protection .............................................................................................................. 31
ESD Field Service kit ................................................................................................................................................. 32
Transporting sensitive components ........................................................................................................................ 33
After working inside your computer ........................................................................................................................ 33
BitLocker ....................................................................................................................................................................... 33
Recommended tools ......................................................................................................................................................... 34
Screw list ............................................................................................................................................................................. 34
Major components of Vostro 15 3530 ......................................................................................................................... 35
Chapter 5: Removing and installing Customer Replaceable Units (CRUs) .................................... 38
Base cover .......................................................................................................................................................................... 38
Removing the base cover .......................................................................................................................................... 38
Installing the base cover ........................................................................................................................................... 40
Memory module ................................................................................................................................................................. 42
Removing the memory module ............................................................................................................................... 42
Installing the memory module .................................................................................................................................. 43
M.2 solid state drive ......................................................................................................................................................... 43
Removing the M.2 2230 solid-state drive ............................................................................................................ 43
Installing the M.2 2230 solid-state drive .............................................................................................................. 44
Removing the M.2 2280 solid-state drive ............................................................................................................ 45
Installing the M.2 2280 solid-state drive .............................................................................................................. 46
Wireless card ...................................................................................................................................................................... 47
Removing the wireless card ..................................................................................................................................... 47
Installing the wireless card ....................................................................................................................................... 48
Speakers .............................................................................................................................................................................. 50
Removing the speakers ............................................................................................................................................ 50
Installing the speakers ............................................................................................................................................... 51
Fan........................................................................................................................................................................................ 53
Removing the fan ....................................................................................................................................................... 53
Installing the fan ......................................................................................................................................................... 54
Chapter 6: Removing and installing Field Replaceable Units (FRUs) ............................................ 55
Battery ................................................................................................................................................................................. 55
Rechargeable Li-ion battery precautions .............................................................................................................. 55
Removing the 3-cell battery .................................................................................................................................... 55
Installing the 3-cell battery ...................................................................................................................................... 56
Removing the 4-cell battery .................................................................................................................................... 57
Installing the 4-cell battery ...................................................................................................................................... 58
Disconnecting the battery cable ............................................................................................................................ 59
Connecting the battery cable .................................................................................................................................. 61
Heat sink .............................................................................................................................................................................. 62
Removing the heat sink - integrated ..................................................................................................................... 62
Installing the heat sink .............................................................................................................................................. 63
Removing the heat sink - discrete ......................................................................................................................... 64
Installing the heat sink - discrete ........................................................................................................................... 65
Touchpad ............................................................................................................................................................................. 66
Removing the touchpad ........................................................................................................................................... 66
Installing the touchpad .............................................................................................................................................. 67
I/O daughter-board cable .............................................................................................................................................. 68
4 Contents

Removing the I/O daughter-board cable .............................................................................................................. 68
Installing the I/O daughter-board cable ................................................................................................................. 69
I/O daughter-board .......................................................................................................................................................... 70
Removing the I/O daughter-board ........................................................................................................................ 70
Installing the I/O daughter-board ........................................................................................................................... 71
Power button ...................................................................................................................................................................... 72
Removing the power button .................................................................................................................................... 72
Installing the power button ...................................................................................................................................... 73
Power button with optional fingerprint reader ........................................................................................................... 74
Removing the power button with optional fingerprint reader ......................................................................... 74
Installing the power button with optional fingerprint reader ........................................................................... 75
Power-adapter port .......................................................................................................................................................... 76
Removing the power-adapter port ........................................................................................................................ 76
Installing the power-adapter port ........................................................................................................................... 77
Display assembly ................................................................................................................................................................ 78
Removing the display assembly .............................................................................................................................. 78
Installing the display assembly ................................................................................................................................ 80
Display hinge caps ............................................................................................................................................................. 83
Removing the display hinge caps ........................................................................................................................... 83
Installing the display hinge caps ............................................................................................................................. 85
Display bezel ....................................................................................................................................................................... 87
Removing the display bezel ...................................................................................................................................... 87
Installing the display bezel ....................................................................................................................................... 89
Display hinges ..................................................................................................................................................................... 90
Removing the display hinges ................................................................................................................................... 90
Installing the display hinges ..................................................................................................................................... 92
Display panel ....................................................................................................................................................................... 93
Removing the display panel ..................................................................................................................................... 93
Installing the display panel ....................................................................................................................................... 98
Camera ................................................................................................................................................................................ 101
Removing the camera ................................................................................................................................................ 101
Installing the camera ................................................................................................................................................. 101
Display cable ..................................................................................................................................................................... 102
Removing the display cable .................................................................................................................................... 102
Installing the display cable ...................................................................................................................................... 103
Display back-cover and antenna assembly ................................................................................................................ 104
Removing the display back-cover and antenna assembly .............................................................................. 104
Installing the display back-cover and antenna assembly ................................................................................ 105
System board .................................................................................................................................................................... 107
Removing the system board .................................................................................................................................. 107
Installing the system board .................................................................................................................................... 109
Palm-rest and keyboard assembly ................................................................................................................................ 112
Removing the palm-rest and keyboard assembly .............................................................................................. 112
Installing the palm-rest and keyboard assembly ................................................................................................ 114
Chapter 7: Software ................................................................................................................... 117
Operating system .............................................................................................................................................................. 117
Drivers and downloads .................................................................................................................................................... 117
Contents 5

Chapter 8: BIOS Setup ............................................................................................................... 118
Entering BIOS setup program ....................................................................................................................................... 118
Navigation keys ................................................................................................................................................................. 118
One time boot menu ........................................................................................................................................................ 118
System setup options ...................................................................................................................................................... 119
Updating the BIOS ........................................................................................................................................................... 124
Updating the BIOS in Windows .............................................................................................................................. 124
Updating the BIOS in Linux and Ubuntu .............................................................................................................. 125
Updating the BIOS using the USB drive in Windows ........................................................................................ 125
Updating the BIOS from the F12 One-Time boot menu ................................................................................... 125
System and setup password ......................................................................................................................................... 126
Assigning a System Setup password .................................................................................................................... 126
Deleting or changing an existing system setup password ............................................................................... 127
Clearing BIOS (System Setup) and System passwords ......................................................................................... 127
Chapter 9: Troubleshooting ....................................................................................................... 128
Handling swollen rechargeable Li-ion batteries ........................................................................................................ 128
Locate the Service Tag or Express Service Code of your Dell computer ........................................................ 128
Dell SupportAssist Pre-boot System Performance Check diagnostics .............................................................. 129
Running the SupportAssist Pre-Boot System Performance Check .............................................................. 129
Built-in self-test (BIST) .................................................................................................................................................. 129
M-BIST ......................................................................................................................................................................... 129
LCD Power rail test (L-BIST) ................................................................................................................................. 130
LCD Built-in Self-Test (BIST) ................................................................................................................................. 130
System-diagnostic lights ................................................................................................................................................. 131
Recovering the operating system ................................................................................................................................ 132
Real-Time Clock (RTC Reset) ...................................................................................................................................... 132
Backup media and recovery options ........................................................................................................................... 133
Wi-Fi power cycle ............................................................................................................................................................ 133
Drain residual flea power (perform hard reset) ........................................................................................................ 133
Chapter 10: Getting help and contacting Dell ............................................................................. 134
6 Contents

Views of Vostro 15 3530
Right
1.SD-card slot
Reads from and writes to the SD card. The computer supports the following card types:
●Secure Digital (SD)
●Secure Digital High Capacity (SDHC)
●Secure Digital Extended Capacity (SDXC)
2.USB 2.0 port
Connect devices such as external storage devices and printers. Provides data transfer speeds up to 480 Mbps.
3.Network port
Connect an Ethernet (RJ45) cable from a router or a broadband modem for network or Internet access, with a transfer rate 
of 10/100/1000 Mbps.
4.Universal audio port
Connect headphones or a headset (headphone and microphone combo).
5.Security-cable slot (wedge-shaped)
Connect a security cable to prevent unauthorized movement of your computer.
Left
Figure 1. For computers shipped with a plastic chassis
1.Power-adapter port (primary power)
Connect a power adapter to provide power to your computer and charge the battery.
2.Power and battery-status light
Indicates the power state and battery state of the computer.1
Views of Vostro 15 3530 7

Solid white—Power adapter is connected and the battery is charging.
Solid amber—Battery charge is low or critical.
Off—Battery is fully charged.
NOTE:  On certain computer models, the power and battery-status light are also used for diagnostics. For more 
information, see the Troubleshooting  section in your computer’s Service Manual .
3.HDMI 1.4 port
Connect to a TV, external display or another HDMI-in enabled device. Provides video and audio output.
NOTE:  The maximum resolution supported by HDMI port is 1920 x 1080 at 60 Hz.
4.USB 3.2 Gen 1 port
Connect devices such as external storage devices and printers. Provides data transfer speeds up to 5 Gbps.
5.USB 3.2 Gen 1 (Type-C) port
Connect devices such as external storage devices and printers. Provides data transfer speeds up to 5 Gbps.
Figure 2. For computers shipped with an aluminum chassis or a discrete graphics card installed
1.Power-adapter port (primary power)
Connect a power adapter to provide power to your computer and charge the battery.
2.Power and battery-status light
Indicates the power state and battery state of the computer.
Solid white—Power adapter is connected and the battery is charging.
Solid amber—Battery charge is low or critical.
Off—Battery is fully charged.
NOTE:  On certain computer models, the power and battery-status light are also used for diagnostics. For more 
information, see the Troubleshooting  section in your computer’s Service Manual .
3.HDMI 1.4 port
Connect to a TV, external display or another HDMI-in enabled device. Provides video and audio output.
NOTE:  The maximum resolution supported by HDMI port is 1920 x 1080 at 60 Hz.
4.USB 3.2 Gen 1 port
Connect devices such as external storage devices and printers. Provides data transfer speeds up to 5 Gbps.
5.USB 3.2 Gen 1 (Type-C) port with Power Delivery/DisplayPort
Connect devices such as external storage devices, printers, and external displays.
Supports Power Delivery that enables two-way power supply between devices. Provides up to 15 W power output that 
enables faster charging.
NOTE:  A USB Type-C to DisplayPort adapter (sold separately) is required to connect a DisplayPort device.
8 Views of Vostro 15 3530

Top
1.Privacy shutter (only for computers shipped with an aluminum chassis)
Slide the privacy shutter to cover the camera lens and protect your privacy when the camera is not in use.
2.Power button with optional fingerprint reader
Press to turn on the computer if it is turned off, in sleep state, or in hibernate state.
When the computer is turned on, press the power button to put the computer into sleep state; press and hold the power 
button for ten seconds to force shut-down the computer.
If the power button has a fingerprint reader, place your finger on the power button to log in.
Figure 3. Active area of the fingerprint reader
NOTE:  The highlighted area indicates the actual active fingerprint reader area and the image is for illustration purposes 
only.
NOTE:  You can customize power-button behavior in Windows. For more information, see www.dell.com/support/
manuals .
3.Precision touchpad
Move your finger on the touchpad to move the mouse pointer. Tap to left-click and two fingers tap to right-click.
4.Right-click area
Views of Vostro 15 3530 9

Press to right-click.
5.Left-click area
Press to left-click.
Front
CAUTION:  The maximum operating angle for the display-panel hinge is 135 degrees.
Figure 4. For computers shipped with a plastic chassis
1.Camera
Enables you to video chat, capture photos, and record videos.
2.Camera-status light
Turns on when the camera is in use.
3.Microphone
Provides digital sound input for audio recording, voice calls, and so on.
4.Display 
Provides visual output.
10 Views of Vostro 15 3530

Figure 5. For computers shipped with an aluminum chassis
1.Left microphone
Provides digital sound input for audio recording and voice calls.
2.Privacy shutter
Slide the privacy shutter to cover the camera lens and protect your privacy when the camera is not in use.
3.Camera
Enables you to video chat, capture photos, and record videos.
4.Camera-status light
Turns on when the camera is in use.
5.Right microphone
Provides digital sound input for audio recording and voice calls.
6.Display 
Provides visual output.
Views of Vostro 15 3530 11

Bottom
1.Right speaker
Provides audio output.
2.Air vents
Air vents provide ventilation for your computer. Clogged air vents can cause overheating and can affect your computer's 
performance and potentially cause hardware issues. Keep the air vents clear of obstructions and clean them regularly to 
prevent build-up of dust and dirt. For more information about cleaning air vents, search for articles in Knowledge Base 
Resource at www.dell.com/support .
3.Service Tag label
The Service Tag is a unique alphanumeric identifier that enables Dell service technicians to identify the hardware 
components in your computer and access warranty information.
4.Left speaker
Provides audio output.
Service Tag
The service tag is a unique alphanumeric identifier that allows Dell service technicians to identify the hardware components in 
your computer and access warranty information.
12 Views of Vostro 15 3530

Battery charge and status light
The following table lists the battery charge and status light behavior of your Vostro 15 3530 .
Table 1. Battery charge and status light behavior 
Power Source LED Behavior System Power State Battery Charge Level
AC Adapter Off S0 - S5 Fully Charged
AC Adapter Solid White S0 - S5 < Fully Charged
Battery Off S0 - S5 11-100%
Battery Solid Amber (590+/-3 nm) S0 - S5 < 10%
●S0 (ON) - System is turned on.
●S4 (Hibernate) - The system consumes the least power compared to all other sleep states. The system is almost at an OFF 
state, except for a trickle power. The context data is written to a hard drive.
●S5 (OFF) - The system is in a shutdown state.
Views of Vostro 15 3530 13

Set up your Vostro 15 3530
About this task
NOTE:  The images in this document may differ from your computer depending on the configuration you ordered.
Steps
1.Connect the power adapter and press the power button.
NOTE:  To conserve battery power, the battery might enter power saving mode. Connect the power adapter and press 
the power button to turn on the computer.
2.Finish operating system setup.
For Ubuntu:
Follow the on-screen instructions to complete the setup. For more information about installing and configuring Ubuntu, 
search in the Knowledge Base Resource at www.dell.com/support .
For Windows:
Follow the on-screen instructions to complete the setup. When setting up, Dell recommends that you:
●Connect to a network for Windows updates.
NOTE:  If connecting to a secured wireless network, enter the password for the wireless network access when 
prompted.
●If connected to the internet, sign-in with or create a Microsoft account. If not connected to the internet, create an 
offline account.
●On the Support and Protection  screen, enter your contact details.
3.Locate and use Dell apps from the Windows Start menu—Recommended.2
14 Set up your Vostro 15 3530

Table 2. Locate Dell apps 
Resources Description
Dell Product Registration
Register your computer with Dell.
Dell Help & Support
Access help and support for your computer.
My Dell
Centralized location for key Dell applications, help articles, and other important information about 
your computer. It also notifies you about the warranty status, recommended accessories, and 
software updates if available.
SupportAssist
SupportAssist is the smart technology that keeps your computer running at its best by optimizing 
settings, detecting issues, removing viruses and notifies when you need to make system 
updates. SupportAssist proactively checks the health of your system's hardware and software. 
When an issue is detected, the necessary system state information is sent to Dell to begin 
troubleshooting. SupportAssist is preinstalled on most of the Dell devices running Windows 
operating system. For more information, see SupportAssist for Home PCs User's Guide on 
www.dell.com/serviceabilitytools .
NOTE:  In SupportAssist, click the warranty expiry date to renew or upgrade your warranty.
Dell Update
Updates your computer with critical fixes and latest device drivers as they become available. For 
more information on using Dell Update, search in the Knowledge Base Resource at www.dell.com/
support .
Dell Digital Delivery
Download software applications, which are purchased but not preinstalled on your computer. 
For more information on using Dell Digital Delivery, search in the Knowledge Base Resource at 
www.dell.com/support .
Set up your Vostro 15 3530 15

Specifications of Vostro 15 3530
Dimensions and weight
The following table lists the height, width, depth, and weight of your Vostro 15 3530 .
Table 3. Dimensions and weight 
Description Computer shipped with a plastic chassis Computer shipped with an aluminum 
chassis
Height:
Front height 16.96 mm (0.67 in.) 15.52 mm (0.61 in.)
Rear height 19.00 mm (0.75 in.) 17.50 mm (0.69 in.)
Width 358.50 mm (14.11 in.) 358.50 mm (14.11 in.)
Depth 235.56 mm (9.27 in.) 234.90 mm (9.25 in.)
Weight (minimum) 1.66 kg (3.66 lb)
NOTE:  The weight of your computer 
depends on the configuration ordered 
and manufacturing variability.1.63 kg (3.60 lb)
NOTE:  The weight of your computer 
depends on the configuration ordered 
and manufacturing variability.
Processor
The following table lists the details of the processors supported by your Vostro 15 3530 .
Table 4. Processor 
Description Option one Option two Option three Option four Option five
Processor type 13th Generation Intel 
Core i3-1305U13th Generation Intel 
Core i5-1334U13th Generation 
Intel Core 
i5-1335U13th Generation 
Intel Core 
i7-1355UIntel Processor 
U300
Processor wattage 15 W 15 W 15 W 15 W 15 W
Processor total core 
count5 10 10 10 5
Performance-cores 1 2 2 2 1
Efficient-cores 4 8 8 8 4
Processor total 
thread counts
NOTE:  Intel 
Hyper-Threading 
Technology is 
only available 6 12 12 12 63
16 Specifications of Vostro 15 3530

Table 4. Processor (continued)
Description Option one Option two Option three Option four Option five
on Performance-
cores.
Processor speed 1.20 GHz to 4.50 
GHz0.90 GHz to 4.60 
GHz0.90 GHz to 4.60 
GHz1.20 GHz to 5.00 
GHz0.90 GHz to 
4.40 GHz
Performance-cores frequency
Processor base 
frequency1.60 GHz 1.30 GHz 1.30 GHz 1.70 GHz 1.20 GHz
Maximum turbo 
frequency4.50 GHz 4.60 GHz 4.60 GHz 5.00 GHz 4.40 GHz
Efficient-cores frequency
Processor base 
frequency1.20 GHz 0.90 GHz 0.90 GHz 1.20 GHz 0.90 GHz
Maximum turbo 
frequency3.30 GHz 3.40 GHz 3.40 GHz 3.70 GHz 3.30 GHz
Processor cache 10 MB 12 MB 12 MB 12 MB 8 MB
Integrated graphics Intel UHD Graphics Intel Iris Xe Graphics Intel Iris Xe 
GraphicsIntel Iris Xe 
GraphicsIntel UHD 
Graphics
Chipset
The following table lists the details of the chipset that is supported for your Vostro 15 3530 .
Table 5. Chipset 
Description Option one Option two
Processors 13th Generation Intel Core i3/i5/i7 
processors and Intel Processor U30013th Generation Intel Core i5/i7 
processors
Chipset Integrated with the processor Integrated with the processor
DRAM bus width 64-bit 64-bit
Flash EPROM 32 MB 32 MB
PCIe bus Up to Gen 3 Up to Gen 4
Operating system
Your Vostro 15 3530  supports the following operating systems:
●Windows 11 Pro
●Windows 11 Pro National Academic
●Windows 11 Home
●Windows 11 Pro Downgrade (Windows 10 image factory installed)
●Ubuntu 22.04 LTS, 64-bit
Specifications of Vostro 15 3530 17

Memory
The following table lists the memory specifications of your Vostro 15 3530.
Table 6. Memory specifications 
Description Values
Memory slots Two SODIMM slots
Memory type DDR4
Memory speed ●2666 MHz (for computers shipped with USB 3.2 Gen 1 
Type-C port)
●3200 MHz (for computers shipped with USB 3.2 Gen 
1 Type-C port with DisplayPort Alt Mode 1.4/Power 
Delivery)
Maximum memory configuration 16 GB
Minimum memory configuration 4 GB
Memory size per slot 4 GB, 8 GB and 16 GB
Memory configurations supported For computers shipped with USB 3.2 Gen 1 Type-C port 
●4 GB, 1 x 4 GB, DDR4, 2666 MHz
●8 GB, 2 x 4 GB, DDR4, 2666 MHz, dual-channel
●8 GB, 1 x 8 GB, DDR4, 2666 MHz
●12 GB, 1 x 4 GB + 1 x 8 GB, DDR4, 2666 MHz, dual-channel
●16 GB, 2 x 8 GB, DDR4, 2666 MHz, dual-channel
●16 GB, 1 x 16 GB, DDR4, 2666 MHz
For computers shipped with USB 3.2 Gen 1 Type-C port 
with DisplayPort Alt Mode 1.4/Power Delivery
●4 GB, 1 x 4 GB, DDR4, 3200 MHz
●8 GB, 2 x 4 GB, DDR4, 3200 MHz, dual-channel
●8 GB, 1 x 8 GB, DDR4, 3200 MHz
●12 GB, 1 x 4 GB + 1 x 8 GB, DDR4, 3200 MHz, dual-channel
●16 GB, 2 x 8 GB, DDR4, 3200 MHz, dual-channel
●16 GB, 1 x 16 GB, DDR4, 3200 MHz
External ports
The following table lists the external ports of your Vostro 15 3530.
Table 7. External ports 
Description Values
Network port One Flip-Down RJ45 Ethernet port
USB ports ●One USB 3.2 Gen 1 port
●One USB 2.0 port
●One USB 3.2 Gen 1 Type-C port (for computers shipped 
with a plastic chassis)
●One USB 3.2 Gen 1 Type-C port with DisplayPort Alt 
Mode 1.4/Power Delivery (for computers shipped with an 
aluminum chassis or a discrete graphics card installed)
18 Specifications of Vostro 15 3530

Table 7. External ports (continued)
Description Values
Audio port One universal audio port
Video port ●One HDMI 1.4 port
NOTE:  The maximum resolution supported by HDMI 
port is 1920 x 1080 at 60 Hz.
●One USB 3.2 Gen 1 Type-C port with DisplayPort Alt 
Mode 1.4/Power Delivery (for computers shipped with an 
aluminum chassis or a discrete graphics card installed)
Media-card reader One SD-card slot
Power-adapter port One DC-in port (4.50 mm standard plug)
Security-cable slot One wedge-shaped lock slot
Internal slots
The following table lists the internal slots of your Vostro 15 3530.
Table 8. Internal slots 
Description Values
M.2 ●One M.2 2230/2280 slot for solid-state drive
●One M.2 2230 slot for WLAN, Wi-Fi/Bluetooth
NOTE:  To learn more about the features of different 
types of M.2 cards, search in the Knowledge Base 
Resource at www.dell.com/support .
Ethernet
The following table lists the wired Ethernet Local Area Network (LAN) specifications of your Vostro 15 3530.
Table 9. Ethernet specifications 
Description Values
Model number Integrated Realtek RTL8111H
Transfer rate 10/100/1000 Mbps
Wireless module
The following table lists the Wireless Local Area Network (WLAN) modules that are supported on your Vostro 15 3530.
Table 10. Wireless module specifications 
Description Option one Option two
Model number Realtek RTL8821CE Realtek RTL8852BE
Transfer rate Up to 433 Mbps Up to 1201 Mbps
Specifications of Vostro 15 3530 19

Table 10. Wireless module specifications (continued)
Description Option one Option two
Frequency bands supported 2.40 GHz/5 GHz 2.40 GHz/5 GHz
Wireless standards ●WiFi 802.11a/b/g
●Wi-Fi 4 (WiFi 802.11n)
●Wi-Fi 5 (WiFi 802.11ac)●WiFi 802.11a/b/g
●Wi-Fi 4 (WiFi 802.11n)
●Wi-Fi 5 (WiFi 802.11ac)
●Wi-Fi 6 (WiFi 802.11ax)
Encryption ●64-bit/128-bit WEP
●AES-CCMP
●TKIP●64-bit/128-bit WEP
●AES-CCMP
●TKIP
Bluetooth wireless card Bluetooth 5.0 wireless card Bluetooth 5.3 wireless card
NOTE:  Particular versions of 
Microsoft Windows may not support 
the full Bluetooth 5.3 wireless card 
functionality.
NOTE:  The version of the Bluetooth wireless card may vary depending on the 
operating system that is installed on your computer.
Audio
The following table lists the audio specifications of your Vostro 15 3530 .
Table 11. Audio specifications 
Description Values
Audio controller There are two audio controller options:
●Realtek ALC3204
●Realtek ALC3254
Stereo conversion Supported
Internal audio interface High definition audio interface
External audio interface One universal audio port
Number of speakers Two
Internal-speaker amplifier Supported
External volume controls Keyboard shortcut controls
Speaker output:
Average speaker output 2 W
Peak speaker output 2.5 W
Subwoofer output Not supported
Microphone ●Single integrated microphone (for computers shipped with 
a plastic chassis)
●Dual-array microphones (for computers shipped with an 
aluminum chassis)
20 Specifications of Vostro 15 3530

Storage
This section lists the storage options on your Vostro 15 3530 .
Your Vostro 15 3530 supports the following storage configurations:
●One M.2 2230/2280 solid-state drive
The M.2 2230/2280 solid-state drive is the primary drive of your computer.
Table 12. Storage specifications 
Storage type Interface type Capacity
M.2 2230, Solid-state drive PCIe NVMe Gen4 x4 QLC Up to 1 TB
M.2 2230, Solid-state drive PCIe NVMe Gen4 x4 Up to 1 TB
M.2 2280, Solid-state drive PCIe NVMe Gen4 x4 Up to 2 TB
Media-card reader
The following table lists the media cards that are supported on your Vostro 15 3530 .
Table 13. Media-card reader specifications 
Description Values
Media-card type One SD-card slot
Media-cards supported ●Secure Digital (SD)
●Secure Digital High Capacity (SDHC)
●Secure Digital Extended Capacity (SDXC)
NOTE:  The maximum capacity supported by the media-card reader varies depending on the standard of the media card 
that is installed on your computer.
Keyboard
The following table lists the keyboard specifications of your Vostro 15 3530 .
Table 14. Keyboard specifications 
Description Values
Keyboard type ●Standard backlit keyboard
●Standard non-backlit keyboard
Keyboard layout QWERTY
Number of keys ●United States and Canada: 99 keys
●United Kingdom: 100 keys
●Japan: 103 keys
Keyboard size X = 18.70 mm key pitch
Y = 18.05 mm key pitch
Keyboard shortcuts Some keys on your keyboard have two symbols on them. 
These keys can be used to type alternate characters or to 
perform secondary functions. To type the alternate character, 
Specifications of Vostro 15 3530 21

Table 14. Keyboard specifications (continued)
Description Values
press Shift and the desired key. To perform secondary 
functions, press Fn and the desired key.
NOTE:  You can define the primary behavior of the 
function keys (F1–F12) by changing Function Key 
Behavior  in BIOS setup program.
Keyboard shortcuts of Vostro 15 3530
NOTE:  Keyboard characters may differ depending on the keyboard language configuration. Keys used for shortcuts remain 
the same across all language configurations.
Some keys on your keyboard have two symbols on them. These keys can be used to type alternate characters or to perform 
secondary functions. The symbol shown on the lower part of the key refers to the character that is typed out when the key is 
pressed. If you press shift and the key, the symbol shown on the upper part of the key is typed out. For example, if you press 2, 
2 is typed out; if you press Shift  + 2, @ is typed out.
The keys F1-F12 at the top row of the keyboard are function keys for multi-media control, as indicated by the icon at the bottom 
of the key. Press the function key to invoke the task represented by the icon. For example, pressing F1 mutes the audio (refer to 
the table below).
However, if the function keys F1-F12 are needed for specific software applications, multi-media functionality can be disabled 
by pressing fn + Esc. Subsequently, multi-media control can be invoked by pressing fn and the respective function key. For 
example, mute audio by pressing fn + f1.
NOTE:  You can also define the primary behavior of the function keys (F1–F12) by changing Function Key Behavior  in 
BIOS setup program.
Table 15. List of keyboard shortcuts 
Function key Primary behavior
F1 Mute audio
F2 Decrease volume
F3 Increase volume
F4 Play/Pause
F5 Click keyboard backlight (optional).
NOTE:  Non-backlight keyboards have F5 function key without the 
backlight icon and do not support toggle keyboard backlight function.
NOTE:  Toggle to cycle the keyboard backlight status through off, low-
backlight, and high-backlight.
F6 Decrease brightness
F7 Increase brightness
F8 Switch to external display
F10 Print screen
F11 Home
F12 End
The fn key is also used with selected keys on the keyboard to invoke other secondary functions.
22 Specifications of Vostro 15 3530

Table 16. Secondary behavior 
Function key Secondary behavior
fn + Esc Toggle fn-key lock
fn + S Toggle scroll lock
fn + B Pause/Break
fn + R System request
fn + Right ctrl Open application menu
fn + /
NOTE:  This option is available only in Brazilian keyboard.Open application menu
fn + Space bar Open emoji menu
fn + T Toggle ultra performance mode
fn + Left arrow Home
fn + Right arrow End
Camera
The following table lists the camera specifications of your Vostro 15 3530 .
Table 17. Camera specifications 
Description Computer shipped with plastic 
chassisComputer shipped with 
aluminum chassis
Number of cameras One One
Camera type There is one camera option:
●HD RGB cameraThere is one camera option:
●FHD RGB camera
Camera location Front camera Front camera
Camera sensor type CMOS sensor technology CMOS sensor technology
Camera resolution:
Still image 0.92 megapixel 2.00 megapixel
Video 1280 x 720 (HD) at 30 fps 1920 x 1080 (FHD) at 30 fps
Diagonal viewing angle 78.60 degrees 82.00 degrees
Touchpad
The following table lists the touchpad specifications of your Vostro 15 3530 . 
Table 18. Touchpad specifications 
Description Values
Touchpad resolution:
Horizontal > 300 dpi
Vertical > 300 dpi
Specifications of Vostro 15 3530 23

Table 18. Touchpad specifications (continued)
Description Values
Touchpad dimensions:
Horizontal 115.00 mm (4.53 in.)
Vertical 80.00 mm (3.15 in.)
Touchpad gestures For more information about touchpad gestures available 
on Windows, see the Microsoft Knowledge Base article at 
support.microsoft.com .
Power adapter
The following table lists the power adapter specifications of your Vostro 15 3530 .
Table 19. Power adapter specifications 
Description Values
Type 65 W
Connector dimensions:
External diameter 4.50 mm (0.18 in.)
Internal diameter 2.90 mm (0.11 in.)
Power-adapter dimensions:
Height 28.00 mm (1.10 in.)
Width 47.00 mm (1.90 in.)
Depth 108.00 mm (4.30 in.)
Power-adapter weight (maximum) 0.29 kg (0.64 lb)
Input voltage 100 VAC - 240 VAC
Input frequency 50 Hz - 60 Hz
Input current (maximum) 1.60 A/1.70 A
Output current (continuous) 3.34 A
Rated output voltage 19.50 VDC
Temperature range:
Operating 0°C to 40°C (32°F to 104°F)
Storage -40°C to 70°C (-40°F to 158°F)
CAUTION:  Operating and storage temperature ranges may differ among components, so operating or storing 
the device outside these ranges may impact the performance of specific components.
24 Specifications of Vostro 15 3530

Battery
The following table lists the battery specifications of your Vostro 15 3530 .
Table 20. Battery specifications 
Description Option one Option two
Battery type 3-cell, 41 Wh, lithium-polymer, 
ExpressCharge4-cell, 54 Wh, lithium-polymer, 
ExpressCharge
Battery voltage 11.25 VDC 15.00 VDC
Battery weight (maximum) 0.19 kg (0.41 lb) 0.24 kg (0.55 lb)
Battery dimensions:
Height 206.40 mm (8.12 in.) 271.90 mm (10.70 in.)
Width 82.00 mm (3.22 in.) 82.00 mm (3.22 in.)
Depth 5.75 mm (0.22 in.) 5.75 mm (0.22 in.)
Temperature range:
Operating When charging battery:
●Initiating charging: 0°C to 45°C 
(32°F to 113°F)
●Upper limit for charging 
(maximum): 50°C (122°F)
When discharging battery:
●Normal discharging: 0°C to 60°C 
(32°F to 140°F)
●Over temperature protection 
(maximum): 70°C (158°F)
NOTE:  The battery does 
not allow discharge past this 
temperature.When charging battery:
●Initiating charging: 0°C to 45°C (32°F 
to 113°F)
●Upper limit for charging (maximum): 
50°C (122°F)
When discharging battery:
●Normal discharging: 0°C to 60°C 
(32°F to 140°F)
●Over temperature protection 
(maximum): 70°C (158°F)
NOTE:  The battery does not allow 
discharge past this temperature.
Storage –20°C to 60°C (-4°F to 140°F) –20°C to 60°C (-4°F to 140°F)
Battery operating time Varies depending on operating 
conditions and can significantly 
reduce under certain power-intensive 
conditions.Varies depending on operating conditions 
and can significantly reduce under certain 
power-intensive conditions.
Battery charging time (approximate)
NOTE:  Control the charging time, 
duration, start and end time, and so 
on, using the Dell Power Manager 
application. For more information 
about Dell Power Manager, search 
in the Knowledge Base Resource at 
www.dell.com/support .Standard Charge/Predominately 
AC User Charge Method: 
●0 - 15°C maximum allowable 
charge time from 0 to 100% RSOC 
is 4 hours
●16 - 50°C maximum allowable 
charge time from 0 to 100% RSOC 
is 3 hours
Express Charge Method: 
●0 - 15°C maximum allowable 
charge time from 0 to 100% RSOC 
is 4 hours
●16 - 45°C normal express charge
●46 - 50°C maximum allowable 
charge time from 0 to 80% RSOC Standard Charge/Predominately AC 
User Charge Method: 
●0 - 15°C maximum allowable charge 
time from 0 to 100% RSOC is 4 hours
●16 - 50°C maximum allowable charge 
time from 0 to 100% RSOC is 3 hours
Express Charge Method: 
●0 - 15°C maximum allowable charge 
time from 0 to 100% RSOC is 4 hours
●16 - 45°C normal express charge
●46 - 50°C maximum allowable charge 
time from 0 to 80% RSOC is 1 hours 
charge time from 0 to 100% RSOC is 2 
hours
Specifications of Vostro 15 3530 25

Table 20. Battery specifications (continued)
Description Option one Option two
is 1 hours charge time from 0 to 
100% RSOC is 2 hours
Express Charge Boost Charge 
Method (Fast Charge for Initial 
35%): 
●16 - 45°C target charge time from 
0 to 35% RSOC is 20 minsExpress Charge Boost Charge Method 
(Fast Charge for Initial 35%): 
●16 - 45°C target charge time from 0 to 
35% RSOC is 20 mins
Coin-cell battery Not supported Not supported
CAUTION:  Operating and storage temperature ranges may differ among components, so operating or storing 
the device outside these ranges may impact the performance of specific components.
CAUTION:  Dell Technologies recommends that you charge the battery regularly for optimal power 
consumption. If your battery charge is depleted, connect the power adapter, turn on your computer, and then 
restart your computer to reduce the power consumption.
Display
The following table lists the display specifications of your Vostro 15 3530 .
Table 21. Display specifications 
Description Option one Option two
Display type 15.6-inch High Definition (HD) 15.6-inch Full High Definition (FHD)
Touch options No No
Display-panel technology Twisted Nematic (TN) In-Plane Switching (IPS)
Display-panel dimensions (active area):
Height 193.54 mm (7.62 in.) 193.59 mm (7.62 in.)
Width 344.23 mm (13.55 in.) 344.14 mm (13.55 in.)
Diagonal 394.91 mm (15.55 in.) 394.87 mm (15.55 in.)
Display-panel native resolution 1366 x 768 1920 x 1080
Luminance (typical) 220 nits 250 nits
Megapixels 1.05 2.07
Color gamut 45% NTSC 45% NTSC
Pixels Per Inch (PPI) 100 141
Contrast ratio (minimum) 400:1 600:1
Response time (maximum) 20 ms 35 ms
Refresh rate 60 Hz 120 Hz
Horizontal view angle 40 +/- degrees (min.) 80 +/- degrees (min.)
26 Specifications of Vostro 15 3530

Table 21. Display specifications (continued)
Description Option one Option two
Vertical view angle 10 (Top)/30 (Bottom) degrees (min.) 80 +/- degrees (min.)
Pixel pitch 0.252 mm x 0.252 mm 0.17925 mm x 0.17925 mm
Power consumption (maximum) 4.20 W 5.25 W
Anti-glare vs glossy finish Anti-glare Anti-glare
Fingerprint reader (optional)
The following table lists the specifications of the optional fingerprint reader of your Vostro 15 3530 .
NOTE:  The fingerprint reader is located on the power button.
Table 22. Fingerprint reader specifications 
Description Values
Fingerprint-reader sensor technology Capacitive sensing
Fingerprint-reader sensor resolution 500 dpi
Fingerprint-reader sensor pixel size 108 x 88 pixel
Sensor
The following table lists the sensor of your Vostro 15 3530 .
Table 23. Sensor 
Sensor support
Adaptive Thermal Performance Accelerometer
GPU—Integrated
The following table lists the specifications of the integrated Graphics Processing Unit (GPU) supported by your Vostro 15 3530 .
Table 24. GPU—Integrated 
Controller Memory size Processor
Intel UHD Graphics Shared system memory 13th Generation Intel Core i3 processors 
and Intel Processor U300
Intel Iris Xe Graphics Shared system memory (dual-channel) 13th Generation Intel Core i5/i7 
processors
Specifications of Vostro 15 3530 27

GPU—Discrete
The following table lists the specifications of the discrete Graphics Processing Unit (GPU) supported by your Vostro 15 3530 .
Table 25. GPU—Discrete 
Controller Memory size Memory type
NVIDIA GeForce MX550 2 GB GDDR6
Hardware security
The following table lists the hardware security of your Vostro 15 3530 .
Table 26. Hardware security  
Hardware security
Windows 10 Hello - Fingerprint Reader (optional)
Trusted Platform Module TPM 2.0
Wedge-shaped lock
Mechanical camera shutter (only for computers shipped with an aluminum chassis)
Operating and storage environment
This table lists the operating and storage specifications of your Vostro 15 3530 .
Airborne contaminant level:  G1 as defined by ISA-S71.04-1985
Table 27. Computer environment  
Description Operating Storage
Temperature range 0°C to 35°C (32°F to 95°F) -40°C to 65°C (-40°F to 149°F)
Relative humidity (maximum) 10% to 90% (non-condensing) 0% to 95% (non-condensing)
Vibration (maximum)*0.66 GRMS 1.30 GRMS
Shock (maximum) 110 G† 160 G†
Altitude range 0 m to 3048 m (0 ft to 10000 ft) 0 m to 10668 m (0 ft to 35000 ft)
CAUTION:  Operating and storage temperature ranges may differ among components, so operating or storing 
the device outside these ranges may impact the performance of specific components.
* Measured using a random vibration spectrum that simulates the user environment.
† Measured using a 2 ms half-sine pulse.
Dell support policy
For information about Dell support policy, search in the Knowledge Base Resource at www.dell.com/support .
28 Specifications of Vostro 15 3530

ComfortView
WARNING:  Prolonged exposure to blue light from the display may lead to long-term effects such as eye strain, 
eye fatigue, or damage to the eyes.
Blue light is a color in the light spectrum which has a short wavelength and high energy. Chronic exposure to blue light, 
particularly from digital sources, may disrupt sleep patterns and cause long-term effects such as eye strain, eye fatigue, or 
damage to the eyes.
ComfortView mode can be enabled and configured using the Dell CinemaColor application.
ComfortView mode complies with TÜV Rheinland's requirement for low blue light software solution displays.
Low blue light: Dell ComfortView software technology reduces harmful blue light emissions to make extended screen time easy 
on your eyes.
To reduce the risk of eye strain, it is also recommended that you:
●Position the display at a comfortable viewing distance between 20 and 28 inches (50 and 70 cm) from your eyes.
●Blink frequently to moisten your eyes, wet your eyes with water, or apply suitable eye drops.
●Look away from your display, and gaze at a distant object at 20 ft (609.60 cm) away for at least 20 seconds during each 
break.
●Take an extended break for 20 minutes every two hours.
Specifications of Vostro 15 3530 29

Working inside your computer
Safety instructions
Use the following safety guidelines to protect your computer from potential damage and to ensure your personal safety. Unless 
otherwise noted, each procedure in this document assumes that you have read the safety information that shipped with your 
computer.
WARNING:  Before working inside your computer, read the safety information that is shipped with your 
computer. For more safety best practices, see the Regulatory Compliance home page at www.dell.com/
regulatory_compliance .
WARNING:  Disconnect your computer from all power sources before opening the computer cover or panels. 
After you finish working inside the computer, replace all covers, panels, and screws before connecting your 
computer to an electrical outlet.
CAUTION:  To avoid damaging the computer, ensure that the work surface is flat, dry, and clean.
CAUTION:  To avoid damaging the components and cards, handle them by their edges, and avoid touching the 
pins and the contacts.
CAUTION:  You should only perform troubleshooting and repairs as authorized or directed by the Dell technical 
assistance team. Damage due to servicing that is not authorized by Dell is not covered by your warranty. See the 
safety instructions that is shipped with the product or at www.dell.com/regulatory_compliance .
CAUTION:  Before touching anything inside your computer, ground yourself by touching an unpainted metal 
surface, such as the metal at the back of the computer. While you work, periodically touch an unpainted metal 
surface to dissipate static electricity which could harm internal components.
CAUTION:  When you disconnect a cable, pull it by its connector or its pull tab, not the cable itself. Some cables 
have connectors with locking tabs or thumbscrews that you must disengage before disconnecting the cable. 
When disconnecting cables, keep them evenly aligned to avoid bending the connector pins. When connecting 
cables, ensure that the ports and the connectors are correctly oriented and aligned.
CAUTION:  Press and eject any installed card from the media-card reader.
CAUTION:  Exercise caution when handling rechargeable Li-ion batteries in laptops. Swollen batteries should not 
be used and should be replaced and disposed properly.
NOTE:  The color of your computer and certain components may differ from what is shown in this document.
Before working inside your computer
Steps
1.Save and close all open files and exit all open applications.
2.Shut down your computer. For Windows operating system, click Start > 
 Power  > Shut down .
NOTE:  If you are using a different operating system, see the documentation of your operating system for shut-down 
instructions.
3.Disconnect your computer and all attached devices from their electrical outlets.
4.Disconnect all attached network devices and peripherals, such as keyboard, mouse, and monitor from your computer.4
30 Working inside your computer

5.Remove any media card and optical disk from your computer, if applicable.
6.Enter the service mode, if you are able to power on your computer.
Service Mode
Service Mode is used to cut-off power, without disconnecting battery cable from system board prior conducting repairs in 
the computer.
CAUTION:  If you are unable to turn on the computer to put it into Service Mode or the computer does not 
support Service Mode then proceed to disconnect the battery cable. To disconnect the battery cable, follow 
the steps in Removing the battery .
NOTE:  Ensure that your computer is shut down and the AC adapter is disconnected.
a.Hold  <B>  key on the keyboard and press the power button for 3 seconds or until the Dell logo appears on the screen.
b.Press any key to continue.
c.If the AC adapter is not disconnected, a message prompting you to remove the AC adapter appears on the screen. 
Remove the AC adapter and then press any key to continue the Service Mode  procedure. The  Service Mode  procedure 
automatically skips the following step if the Owner Tag  of the computer is not set up in advance by the user.
d.When the ready-to-proceed message appears on the screen, press any key to proceed. The computer emits three short 
beeps and shuts down immediately.
e.Once the computer shuts down, it has successfully entered Service Mode.
NOTE:  If you are unable to power on your computer or unable to enter service mode skip this process.
Safety precautions
The safety precautions chapter details the primary steps to be taken before performing any disassembly instructions.
Observe the following safety precautions before you perform any installation or break-fix procedures involving disassembly or 
reassembly:
●Turn off the computer and all attached peripherals.
●Disconnect the computer and all attached peripherals from AC power.
●Disconnect all network cables, telephone, and telecommunications lines from the computer.
●Use an ESD field service kit when working inside any notebook  to avoid electrostatic discharge (ESD) damage.
●After removing any computer component, carefully place the removed component on an anti-static mat.
●Wear shoes with non-conductive rubber soles to reduce the chance of getting electrocuted.
●Unplugging, pressing, and holding the power button for 15 seconds should discharge residual power in the system board.
Standby power
Dell products with standby power must be unplugged before you open the case. Systems that incorporate standby power are 
powered while turned off. The internal power enables the computer to be remotely turned on (Wake-on-LAN) and suspended 
into a sleep mode and has other advanced power management features.
Bonding
Bonding is a method for connecting two or more grounding conductors to the same electrical potential. This is done by using a 
field service electrostatic discharge (ESD) kit. When connecting a bonding wire, ensure that it is connected to bare metal and 
never to a painted or nonmetal surface. The wrist strap should be secure and in full contact with your skin, and ensure that you 
remove all jewelry such as watches, bracelets, or rings prior to bonding yourself and the equipment.
Electrostatic discharge—ESD protection
ESD is a major concern when you handle electronic components, especially sensitive components such as expansion cards, 
processors, memory modules, and system boards. Slight charges can damage circuits in ways that may not be obvious, such 
as intermittent problems or a shortened product life span. As the industry pushes for lower power requirements and increased 
density, ESD protection is an increasing concern.
Working inside your computer 31

Due to the increased density of semiconductors used in recent Dell products, the sensitivity to static damage is now higher than 
in previous Dell products. For this reason, some previously approved methods of handling parts are no longer applicable.
Two recognized types of ESD damage are catastrophic and intermittent failures.
●Catastrophic  – Catastrophic failures represent approximately 20 percent of ESD-related failures. The damage causes an 
immediate and complete loss of device functionality. An example of catastrophic failure is a memory DIMM that has received 
a static shock and immediately generates a "No POST/No Video" symptom with a beep code that is emitted for missing or 
nonfunctional memory.
●Intermittent  – Intermittent failures represent approximately 80 percent of ESD-related failures. The high rate of 
intermittent failures means that most of the time when damage occurs, it is not immediately recognizable. The DIMM 
receives a static shock, but the tracing is merely weakened and does not immediately produce outward symptoms that are 
related to the damage. The weakened trace may take weeks or months to melt, and in the meantime may cause degradation 
of memory integrity, intermittent memory errors, and so on.
The more difficult type of damage to recognize and troubleshoot is the intermittent (also called latent or "walking wounded") 
failure.
Perform the following steps to prevent ESD damage:
●Use a wired ESD wrist strap that is properly grounded. Wireless anti-static straps do not provide adequate protection. 
Touching the chassis before handling parts does not ensure adequate ESD protection on parts with increased sensitivity to 
ESD damage.
●Handle all static-sensitive components in a static-safe area. If possible, use anti-static floor pads and workbench pads.
●When unpacking a static-sensitive component from its shipping carton, do not remove the component from the anti-static 
packing material until you are ready to install the component. Before unwrapping the anti-static packaging, ensure that you 
discharge static electricity from your body.
●Before transporting a static-sensitive component, place it in an anti-static container or packaging.
ESD Field Service kit
The unmonitored Field Service kit is the most commonly used service kit. Each Field Service kit includes three main components: 
anti-static mat, wrist strap, and bonding wire.
Components of an ESD field service kit
The components of an ESD field service kit are:
●Anti-Static Mat  – The anti-static mat is dissipative and parts can be placed on it during service procedures. When using an 
anti-static mat, your wrist strap should be snug and the bonding wire should be connected to the mat and to any bare metal 
on the computer being worked on. Once deployed properly, service parts can be removed from the ESD bag and placed 
directly on the mat. ESD-sensitive items are safe in your hand, on the ESD mat, in the computer, or inside an ESD bag.
●Wrist Strap and Bonding Wire  – The wrist strap and bonding wire can be either directly connected between your wrist 
and bare metal on the hardware if the ESD mat is not required, or connected to the anti-static mat to protect hardware that 
is temporarily placed on the mat. The physical connection of the wrist strap and bonding wire between your skin, the ESD 
mat, and the hardware is known as bonding. Use only Field Service kits with a wrist strap, mat, and bonding wire. Never 
use wireless wrist straps. Always be aware that the internal wires of a wrist strap are prone to damage from normal wear 
and tear, and must be checked regularly with a wrist strap tester in order to avoid accidental ESD hardware damage. It is 
recommended to test the wrist strap and bonding wire at least once per week.
●ESD Wrist Strap Tester  – The wires inside an ESD strap are prone to damage over time. When using an unmonitored kit, 
it is a best practice to regularly test the strap prior to each service call, and at a minimum, test once per week. A wrist strap 
tester is the best method for doing this test. If you do not have your own wrist strap tester, check with your regional office 
to find out if they have one. To perform the test, plug the bonding-wire of wrist-strap into the tester while it is strapped to 
your wrist and push the button to test. A green LED is lit if the test is successful; a red LED is lit and an alarm sounds if the 
test fails.
●Insulator Elements – It is critical to keep ESD sensitive devices, such as plastic heat sink casings, away from internal parts 
that are insulators and often highly charged.
●Working Environment – Before deploying the ESD Field Service kit, assess the situation at the customer location. For 
example, deploying the kit for a server environment is different than for a desktop or laptop environment. Servers are 
typically installed in a rack within a data center; desktops or laptops are typically placed on office desks or cubicles. Always 
look for a large open flat work area that is free of clutter and large enough to deploy the ESD kit with additional space to 
accommodate the type of computer that is being repaired. The workspace should also be free of insulators that can cause an 
ESD event. On the work area, insulators such as Styrofoam and other plastics should always be moved at least 12 inches or 
30 centimeters away from sensitive parts before physically handling any hardware components.
32 Working inside your computer

●ESD Packaging – All ESD-sensitive devices must be shipped and received in static-safe packaging. Metal, static-shielded 
bags are preferred. However, you should always return the damaged part using the same ESD bag and packaging that the 
new part arrived in. The ESD bag should be folded over and taped shut and all the same foam packing material should be 
used in the original box that the new part arrived in. ESD-sensitive devices should be removed from packaging only at an 
ESD-protected work surface, and parts should never be placed on top of the ESD bag because only the inside of the bag is 
shielded. Always place parts in your hand, on the ESD mat, in the computer, or inside an anti-static bag.
●Transporting Sensitive Components  – When transporting ESD sensitive components such as replacement parts or parts 
to be returned to Dell, it is critical to place these parts in anti-static bags for safe transport.
ESD protection summary
It is recommended to always use the traditional wired ESD grounding wrist strap and protective anti-static mat when servicing 
Dell products. In addition, it is critical to keep sensitive parts separate from all insulator parts while performing service and use 
anti-static bags for transporting sensitive components.
Transporting sensitive components
When transporting ESD sensitive components such as replacement parts or parts to be returned to Dell, it is critical to place 
these parts in anti-static bags for safe transport.
After working inside your computer
About this task
CAUTION:  Leaving stray or loose screws inside your computer may severely damage your computer.
Steps
1.Replace all screws and ensure that no stray screws remain inside your computer.
2.Connect any external devices, peripherals, or cables you removed before working on your computer.
3.Replace any media cards, discs, or any other parts that you removed before working on your computer.
4.Connect your computer and all attached devices to their electrical outlets.
NOTE:  To exit service mode, ensure to connect the AC adapter to the power-adapter port on your computer.
5.Press the power button to turn on the computer. Your computer will automatically return to normal functioning mode.
BitLocker
CAUTION:  If BitLocker is not suspended before updating the BIOS, the next time you reboot the system it will 
not recognize the BitLocker key. You will then be prompted to enter the recovery key to progress, and the 
system will ask for this on each reboot. If the recovery key is not known this can result in data loss or an 
unnecessary operating system reinstall. For more information about this subject, For information on updating 
the BIOS on Dell systems with BitLocker enabled , search in the Knowledge Base Resource at www.dell.com/
support .
CAUTION:  If BitLocker is not suspended, replacing parts like system board or storage drives on a system with 
BitLocker / Device encryption enabled will also trigger the encryption and lock the user out. The recovery 
key is required to regain access to Windows. The recovery key is automatically saved to the user's Microsoft 
Account (MSA) when the device is encrypted and can be retrieved from https://account.microsoft.com/
devices/recoverykey
For more information, refer to Windows Support article: Finding your BitLocker recovery key in Windows
Working inside your computer 33

Recommended tools
The procedures in this document may require the following tools:
●Phillips screwdriver #0
●Plastic scribe
Screw list
NOTE:  When removing screws from a component, it is recommended to note the screw type, and the quantity of screws, 
and then place them in a screw storage box. This is to ensure that the correct number of screws and correct screw type is 
restored when the component is replaced.
NOTE:  Some computers have magnetic surfaces. Ensure that the screws are not left attached to such surfaces when 
replacing a component.
NOTE:  Screw color may vary with the configuration ordered.
Table 28. Screw list 
Component Screw type Quantity Screw image
Base cover ●M2x5
●Captive screw
NOTE:  Screws are 
part of the base 
cover.●6
●2
●
●
Battery M2x3 ●3-cell battery: 3
●4-cell battery: 4
M.2 2230/2280 solid-state 
driveM2x2 1
Wireless-card bracket M2x3.5 1
Fan M2x5 2
Heat sink - integrated Captive screw
NOTE:  Screws are 
part of the integrated 
heat sink.4
Heat sink - discrete ●Captive screw
NOTE:  Screws 
are part of the 
discrete heat sink.
●M2x3●4
●3●
●
34 Working inside your computer

Table 28. Screw list (continued)
Component Screw type Quantity Screw image
Touchpad bracket M2x2 3
Touchpad M2x2 2
I/O daughter-board M2x3.5 2
Power button M2x2 1
Power button with optional 
fingerprint readerM2x2 1
Display assembly M2.5x5 4
Display hinges M2.5x3.8 6
System board ●M2x2
●M2x3.5●1
●4
●
●
RF metal wall bracket
NOTE:  Only for computers 
shipped with an aluminum 
chassis and a 4-cell battery 
installed.M2x2 2
Major components of Vostro 15 3530
The following image shows the major components of Vostro 15 3530 .
Working inside your computer 35

1.Base cover 2.Battery
3.Speakers 4.Power button with optional fingerprint reader
5.Touchpad 6.Touchpad bracket
7.Power adapter port 8.M.2 2280 solid-state drive thermal plate
36 Working inside your computer

9.M.2 2280 solid-state drive 10.Palm-rest and keyboard assembly
11.Display assembly 12.Memory module
13.Wireless card 14.I/O daughter-board
15.System board 16.Heat sink
17.Fan 18.I/O daughter-board cable
NOTE:  Dell provides a list of components and their part numbers for the original system configuration purchased. These 
parts are available according to warranty coverages purchased by the customer. Contact your Dell sales representative for 
purchase options.
Working inside your computer 37

Removing and installing Customer 
Replaceable Units (CRUs)
The replaceable components in this chapter are Customer Replaceable Units (CRUs).
CAUTION:  Customers can replace only the Customer Replaceable Units (CRUs) following the safety precautions 
and replacement procedures.
NOTE:  The images in this document may differ from your computer depending on the configuration you ordered.
Base cover
Removing the base cover
Prerequisites
1.Follow the procedure in Before working inside your computer .
NOTE:  Ensure that your computer is in Service Mode. For more information see, step 6 in Before working inside your 
computer .
About this task
The following images indicate the location of the base cover and provide a visual representation of the removal procedure.5
38 Removing and installing Customer Replaceable Units (CRUs)

Removing and installing Customer Replaceable Units (CRUs) 39

Steps
1.Remove the six screws (M2x5) and loosen the two captive screws that secure the base cover to the palm-rest and 
keyboard assembly.
2.Using a plastic scribe, pry open the base cover starting from the recesses, which are located in the U-shaped indents at the 
top edge of the base cover, near the hinges.
3.Pry open the top side of the base cover and continue working on the left and right sides to open the base cover.
4.Lift and remove the base cover off the palm-rest and keyboard assembly.
Installing the base cover
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following images indicate the location of the base cover and provide a visual representation of the installation procedure.
40 Removing and installing Customer Replaceable Units (CRUs)

Removing and installing Customer Replaceable Units (CRUs) 41

Steps
1.Place the base cover on top of the palm-rest and keyboard assembly.
2.Align the screw holes on the base cover with the screw holes on the palm-rest and keyboard assembly, and snap the base 
cover latches into place.
3.Tighten the two captive screws and replace the six screws (M2x5) to secure the base cover to the palm-rest and keyboard 
assembly.
Next steps
1.Follow the procedure in After working inside your computer .
Memory module
Removing the memory module
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
About this task
The following images indicate the location of the memory module and provide a visual representation of the removal procedure.
Steps
1.Using a plastic scribe, carefully pry the memory-module retention clips away from the memory module until the memory 
module pops-up.
2.Slide the memory module at an angle and remove it from the memory-module slot on the system board.
3.Repeat step 1 and 2 for the second memory module, if installed.
42 Removing and installing Customer Replaceable Units (CRUs)

Installing the memory module
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following images indicate the location of the memory module and provide a visual representation of the installation 
procedure.
Steps
1.Align the notch on the memory module with the tab on the memory-module slot on the system board.
2.Slide the memory module firmly into the memory-module slot at an angle.
3.Press down on the memory module to snap it on the system board. Ensure that the memory-module retention clips have 
locked the memory module.
NOTE:  If you do not hear the click, remove the memory module and reinstall it.
4.Repeat steps 1, 2 and 3 to install the second memory module, if applicable.
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
M.2 solid state drive
Removing the M.2 2230 solid-state drive
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
Removing and installing Customer Replaceable Units (CRUs) 43

About this task
NOTE:  This procedure applies only to computers shipped with a M.2 2230 solid-state drive installed.
NOTE:  The M.2 card that is installed on your system depends on the configuration ordered. Supported card configurations 
on the M.2 card slot are:
●M.2 2230 solid-state drive
●M.2 2280 solid-state drive
The following images indicate the location of the M.2 2230 solid-state drive and provide a visual representation of the removal 
procedure.
Steps
1.Remove the screw (M2x2) that secures the M.2 2230 solid-state drive thermal plate to the palm-rest and keyboard 
assembly.
2.Slide and remove the M.2 2230 solid-state drive thermal plate from the M.2 2230 solid-state drive.
3.Slide and remove the M.2 2230 solid-state drive from the M.2 card slot on the system board.
Installing the M.2 2230 solid-state drive
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following images indicate the location of the M.2 2230 solid-state drive and provide a visual representation of the 
installation procedure.
44 Removing and installing Customer Replaceable Units (CRUs)

Steps
1.Align the notch on the M.2 2230 solid-state drive with the tab on the M.2 card slot on the system board.
2.Slide the M.2 2230 solid-state drive in the M.2 card slot on the system board.
3.Place the M.2 2230 solid-state drive thermal plate on the M.2 2230 solid-state drive.
NOTE:  When installing the M.2 2230 solid-state drive thermal plate on the system, tuck the tabs on the M.2 2230 
solid-state drive thermal plate under the hooks on the palm-rest and keyboard assembly.
4.Align the screw hole on the M.2 2230 solid-state drive thermal plate with the screw hole on the palm-rest and keyboard 
assembly.
5.Replace the screw (M2x2) to secure the M.2 2230 solid-state drive thermal plate to the palm-rest and keyboard assembly.
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
Removing the M.2 2280 solid-state drive
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
About this task
NOTE:  This procedure applies only to computers shipped with a M.2 2280 solid-state drive installed.
NOTE:  The M.2 card that is installed on your system depends on the configuration ordered. Supported card configurations 
on the M.2 card slot are:
●M.2 2230 solid-state drive
●M.2 2280 solid-state drive
The following images indicate the location of the M.2 2280 solid-state drive and provide a visual representation of the removal 
procedure.
Removing and installing Customer Replaceable Units (CRUs) 45

Steps
1.Remove the screw (M2x2) that secures the M.2 2280 solid-state drive thermal plate to the palm-rest and keyboard 
assembly.
2.Slide and remove the M.2 2280 solid-state drive thermal plate from the palm-rest and keyboard assembly.
3.Slide and remove the M.2 2280 solid-state drive from the M.2 card slot on the system board.
Installing the M.2 2280 solid-state drive
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following images indicate the location of the M.2 2280 solid-state drive and provide a visual representation of the 
installation procedure.
46 Removing and installing Customer Replaceable Units (CRUs)

Steps
1.Align the notch on the M.2 2280 solid-state drive with the tab on the M.2 card slot on the system board.
2.Slide the M.2 2280 solid-state drive in the M.2 card slot on the system board.
3.Place the M.2 2280 solid-state drive thermal plate on the M.2 2280 solid-state drive.
NOTE:  When installing the M.2 2280 solid-state drive thermal plate on the computer, tuck the tabs on the M.2 2280 
solid-state drive thermal plate under the hooks on the palm-rest and keyboard assembly.
4.Align the screw hole on the M.2 2280 solid-state drive thermal plate with the screw hole on the palm-rest and keyboard 
assembly.
5.Replace the screw (M2x2) to secure the M.2 2280 solid-state drive thermal plate to the palm-rest and keyboard assembly.
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
Wireless card
Removing the wireless card
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
About this task
The following images indicate the location of the wireless card and provide a visual representation of the removal procedure.
Removing and installing Customer Replaceable Units (CRUs) 47

Steps
1.Remove the screw (M2x3.5) that secures the wireless-card bracket to the system board.
2.Lift the wireless-card bracket off the wireless card.
3.Disconnect the wireless-antenna cables from the connectors on the wireless card.
4.Slide and remove the wireless card from the wireless-card slot on the system board.
Installing the wireless card
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following images indicate the location of the wireless card and provide a visual representation of the installation procedure.
48 Removing and installing Customer Replaceable Units (CRUs)

Steps
1.At an angle, slide the wireless card into the wireless-card slot on the system board.
2.Connect the wireless-antenna cables to the wireless card.
The following table provides the antenna-cable color scheme for the wireless card supported by your computer.
Table 29. Antenna-cable color scheme 
Connector on the 
wireless cardAntenna-cable color Silkscreen marking
Main White MAIN △ (white triangle)
Auxiliary Black AUX ▲ (black triangle)
3.Place the wireless-card bracket on the wireless card.
4.Align the screw hole on the wireless-card bracket with the screw hole on the system board.
5.Replace the screw (M2x3.5) to secure the wireless-card bracket and the wireless card to the system board.
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
Removing and installing Customer Replaceable Units (CRUs) 49

Speakers
Removing the speakers
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
About this task
The following image indicates the location of the speakers and provides a visual representation of the removal procedure.
Steps
1.Disconnect the speaker cable from the connector on the system board.
NOTE:  To disconnect the speaker cable featuring an eight-pin connector from the system board, pry up the bottom side 
of the cable connector’s head first and then pull it away from the connector.
50 Removing and installing Customer Replaceable Units (CRUs)

2.Unroute and remove the speaker cable from the routing guides on palm-rest and keyboard assembly.
3.Lift the speakers, along with the cable, off the palm-rest and keyboard assembly.
Installing the speakers
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  If the rubber grommets are pushed out when removing the speakers, push them back in before replacing the 
speakers.
The following image indicates the location of the speakers and provides a visual representation of the installation procedure.
Removing and installing Customer Replaceable Units (CRUs) 51

Steps
1.Using the alignment posts and rubber grommets, place the speakers in the slots on the palm-rest and keyboard assembly.
NOTE:  Ensure that the alignment posts are threaded through the rubber grommets on the speakers.
2.Route the speaker cable along the bottom side of the palm-rest and keyboard assembly, over the wireless-antenna cables. 
Then secure the speaker cable into the routing guides on the palm-rest and keyboard assembly.
NOTE:  The speaker cable must be routed through the routing guides to avoid damaging the speaker cable when 
installing the base cover.
52 Removing and installing Customer Replaceable Units (CRUs)

3.Connect the speaker cable to the connector on the system board.
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
Fan
Removing the fan
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
About this task
The following image indicates the location of the fan and provides a visual representation of the removal procedure.
Steps
1.Disconnect the fan cable from the connector on the system board.
2.Remove the two screws (M2x5) that secure the fan to the palm-rest and keyboard assembly.
3.Lift the fan off the palm-rest and keyboard assembly.
Removing and installing Customer Replaceable Units (CRUs) 53

Installing the fan
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following image indicates the location of the fan and provides a visual representation of the installation procedure.
Steps
1.Place the fan in the slot on the palm-rest and keyboard assembly.
2.Align the screw holes on the fan to the screw holes on the palm-rest and keyboard assembly.
3.Replace the two screws (M2x5) to secure the fan to the palm-rest and keyboard assembly.
4.Connect the fan cable to the connector on the system board.
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
54 Removing and installing Customer Replaceable Units (CRUs)

Removing and installing Field Replaceable 
Units (FRUs)
The replaceable components in this chapter are Field Replaceable Units (FRUs).
CAUTION:  The information in this removing and installing FRU's section is intended for authorized service 
technicians only.
CAUTION:  To avoid any potential damage to the component or loss of data, ensure that an authorized service 
technician replaces the Field Replaceable Units (FRUs).
CAUTION:  Dell Technologies recommends that this set of repairs, if needed, to be conducted by trained 
technical repair specialists.
CAUTION:  As a reminder, your warranty does not cover damages that may occur during FRU repairs that are not 
authorized by Dell Technologies.
NOTE:  The images in this document may differ from your computer depending on the configuration you ordered.
Battery
Rechargeable Li-ion battery precautions
CAUTION: 
●Exercise caution when handling rechargeable Li-ion batteries.
●Discharge the battery completely before removing it. Disconnect the AC power adapter from the computer 
and operate the computer solely on battery power—the battery is fully discharged when the computer no 
longer turns on when the power button is pressed.
●Do not crush, drop, mutilate, or penetrate the battery with foreign objects.
●Do not expose the battery to high temperatures, or disassemble battery packs and cells.
●Do not apply pressure to the surface of the battery.
●Do not bend the battery.
●Do not use tools of any kind to pry on or against the battery.
●Ensure any screws during the servicing of this product are not lost or misplaced, to prevent accidental 
puncture or damage to the battery and other computer components.
●If the battery gets stuck inside your computer as a result of swelling, do not try to release it as puncturing, 
bending, or crushing a rechargeable Li-ion battery can be dangerous. In such an instance, contact Dell 
technical support for assistance. See www.dell.com/contactdell .
●Always purchase genuine batteries from www.dell.com  or authorized Dell partners and resellers.
●Swollen batteries should not be used and should be replaced and disposed properly. For guidelines on how to 
handle and replace swollen rechargeable Li-ion batteries, see Handling swollen rechargeable Li-ion batteries .
Removing the 3-cell battery
CAUTION:  The information in this section is intended for authorized service technicians only.6
Removing and installing Field Replaceable Units (FRUs) 55

Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
About this task
The following image indicate s the location of the 3-cell battery and provide s a visual representation of the removal procedure.
Steps
1.Using the pull tab, disconnect the battery cable from the connector on the system board.
2.Remove the three screws (M2x3) that secure the 3-cell battery to the palm-rest and keyboard assembly.
3.Lift the 3-cell battery, along with the battery cable, off the palm-rest and keyboard assembly.
NOTE:  Computers shipped with an aluminum chassis have two RF metal wall brackets installed on the left and right 
sides of the palm-rest and keyboard assembly. The right bracket is secured by the battery with no screw, tape, or 
adhesives otherwise securing the bracket in place. Technicians should take note of whether the bracket has fallen out of 
the computer during subsequent removal procedures.
Installing the 3-cell battery
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following image indicate s the location of the 3-cell battery and provide s a visual representation of the installation procedure.
56 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Place the 3-cell battery, along with the battery cable, in the slot on the palm-rest and keyboard assembly.
2.Align the screw holes on the 3-cell battery to the screw holes on the palm-rest and keyboard assembly.
3.Replace the three screws (M2x3) to secure the 3-cell battery to the palm-rest and keyboard assembly.
4.Connect the battery cable to the connector on the system board.
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
Removing the 4-cell battery
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
About this task
The following image indicates the location of the 4-cell battery and provides a visual representation of the removal procedure.
Removing and installing Field Replaceable Units (FRUs) 57

Steps
1.Using the pull tab, disconnect the battery cable from the connector on the system board.
2.Remove the four screws (M2x3) that secure the 4-cell battery to the palm-rest and keyboard assembly.
3.Lift the 4-cell battery, along with the battery cable, off the palm-rest and keyboard assembly.
NOTE:  Computers shipped with an aluminum chassis have two RF metal wall brackets installed on the left and right 
sides of the palm-rest and keyboard assembly. The right bracket is secured by the battery with no screw, tape, or 
adhesives otherwise securing the bracket in place. As a result, technicians should take note of whether the bracket has 
fallen out of the computer during subsequent removal procedures.
4.Carefully push and remove the battery rubber stopper out of the bottom-left screw hole of the 4-cell battery.
NOTE:  When replacing the 4-cell battery, use a plastic scribe to push upward through the bottom-left screw hole to 
remove the battery rubber stopper and transfer it to the new replacement 4-cell battery.
Installing the 4-cell battery
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following image indicates the location of the 4-cell battery and provides a visual representation of the installation procedure.
58 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Carefully push and insert the battery rubber stopper into the bottom-left screw hole of the 4-cell battery.
NOTE:  When replacing the 4-cell battery, use a plastic scribe to push downward into the bottom-left screw hole to 
insert the battery rubber stopper into the slot on the new replacement 4-cell battery.
2.Place the 4-cell battery, along with the battery cable, in the slot on the palm-rest and keyboard assembly.
3.Align the screw holes on the 4-cell battery to the screw holes on the palm-rest and keyboard assembly.
4.Replace the four screws (M2x3) to secure the 4-cell battery to the palm-rest and keyboard assembly.
5.Connect the battery cable to the connector on the system board.
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
Disconnecting the battery cable
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the 3-cell battery  or the 4-cell battery , whichever applicable.
About this task
The following image indicates the location of the battery cable and provides a visual representation of the removal procedure.
Removing and installing Field Replaceable Units (FRUs) 59

Steps
1.Unroute the battery cable from the routing guides on the battery.
2.Open the latch and disconnect the battery cable from the connector on the battery.
NOTE:  To disconnect the battery cable, first push the latch downward to release the connector, and then pull the 
connector upward to disconnect it from the battery.
60 Removing and installing Field Replaceable Units (FRUs)

CAUTION:  DO NOT pull the battery cable to disconnect it from the battery. This may damage the battery or 
the battery cable.
Connecting the battery cable
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following image indicates the location of the battery cable and provides a visual representation of the installation procedure.
Removing and installing Field Replaceable Units (FRUs) 61

Steps
1.Connect the battery cable to the connector on the battery and close the latch.
2.Route the battery cable through the routing guides on the battery.
Next steps
1.Install the 3-cell battery  or the 4-cell battery , whichever applicable.
2.Install the base cover .
3.Follow the procedure in After working inside your computer .
Heat sink
Removing the heat sink - integrated
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
About this task
NOTE:  The integrated heat sink may become hot during normal operation. Allow sufficient time for the integrated heat sink 
to cool before you touch it.
NOTE:  For maximum cooling of the processor, do not touch the heat transfer areas on the integrated heat sink. The oils in 
your skin can reduce the heat transfer capability of the thermal grease.
The following image indicates the location of the integrated heat sink and provides a visual representation of the removal 
procedure.
62 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Loosen the four captive screws that secure the heat sink to the system board.
NOTE:  Loosen the four captive screws in the reverse sequential order mentioned on the heat sink [4 > 3 > 2 > 1].
2.Lift the heat sink off the system board.
Installing the heat sink
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  If either the system board or the integrated heat sink is replaced, use the thermal grease provided in the kit to 
ensure that thermal conductivity is achieved.
The following image indicates the location of the integrated heat sink and provides a visual representation of the installation 
procedure.
Removing and installing Field Replaceable Units (FRUs) 63

Steps
1.Place the heat sink in the slot on the system board.
2.Align the screw holes on the heat sink to the screw holes on the system board.
3.Tighten the four captive screws to secure the heat sink to the system board.
NOTE:  Tighten the four captive screws in the sequential order mentioned on the heat sink [1 > 2 > 3 > 4].
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
Removing the heat sink - discrete
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
About this task
NOTE:  The discrete heat sink may become hot during normal operation. Allow sufficient time for the discrete heat sink to 
cool before you touch it.
NOTE:  For maximum cooling of the processor, do not touch the heat transfer areas on the discrete heat sink. The oils in 
your skin can reduce the heat transfer capability of the thermal grease.
The following image indicates the location of the discrete heat sink and provides a visual representation of the removal 
procedure.
64 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Remove the three screws (M2x3) and loosen the four captive screws that secure the discrete heat sink to the system 
board.
NOTE:  Loosen the four captive screws in the reverse sequential order mentioned on the discrete heat sink [4 > 3 > 2 > 
1].
2.Lift and remove the discrete heat sink from the system board.
Installing the heat sink - discrete
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  If either the system board or the discrete heat sink is replaced, use the thermal grease provided in the kit to ensure 
that thermal conductivity is achieved.
The following image indicates the location of the discrete heat sink and provides a visual representation of the installation 
procedure.
Removing and installing Field Replaceable Units (FRUs) 65

Steps
1.Place the discrete heat sink on the system board.
2.Align the screw holes on the discrete heat sink to the screw holes on the system board.
3.Replace the three screws (M2x3) and tighten the four captive screws to secure the discrete heat sink to the system board.
NOTE:  Tighten the four captive screws in the sequential order mentioned on the discrete heat sink [1 > 2 > 3 > 4].
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
Touchpad
Removing the touchpad
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the 3-cell battery  or the 4-cell battery , whichever applicable.
About this task
The following images indicate the location of the touchpad and provide a visual representation of the removal procedure.
66 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Open the latch and disconnect the touchpad FFC from the connector on the system board.
2.Remove the three screws (M2x2) that secure the touchpad bracket to the palm-rest and keyboard assembly.
3.Lift the touchpad bracket off the touchpad assembly.
4.Remove the two screws (M2x2) that secure the touchpad to the palm-rest and keyboard assembly.
5.Lift the touchpad, along with the touchpad FFC, off the palm-rest and keyboard assembly.
6.Open the latch and disconnect the touchpad FFC from the connector on the touchpad.
Installing the touchpad
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  Ensure that the touchpad is aligned with the guides available on the palm-rest and keyboard assembly, and the gap 
on either sides of the touchpad is equal.
The following images indicate the location of the touchpad and provide a visual representation of the installation procedure.
Removing and installing Field Replaceable Units (FRUs) 67

Steps
1.Connect the touchpad FFC to the connector on the touchpad and close the latch.
2.Align and place the touchpad, along with the touchpad FFC, in the slot on the palm-rest and keyboard assembly.
3.Replace the two screws (M2x2) to secure the touchpad to the palm-rest and keyboard assembly.
4.Place the touchpad bracket in the slot on the palm-rest and keyboard assembly.
5.Align the screw holes on the touchpad bracket to the screw holes on the palm-rest and keyboard assembly.
6.Replace the three screws (M2x2) to secure the touchpad bracket to the palm-rest and keyboard assembly.
7.Connect the touchpad FFC to the connector on the system board and close the latch.
Next steps
1.Install the 3-cell battery  or the 4-cell battery , whichever applicable.
2.Install the base cover .
3.Follow the procedure in After working inside your computer .
I/O daughter-board cable
Removing the I/O daughter-board cable
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the fan.
About this task
The following image indicates the location of the I/O daughter-board cable and provides a visual representation of the removal 
procedure.
68 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Open the latch and disconnect the I/O daughter-board cable from the connector on the system board.
2.Open the latch and disconnect the I/O daughter-board cable from the connector on the I/O daughter-board.
3.Remove the I/O daughter-board cable from the palm-rest and keyboard assembly.
NOTE:  For computers shipped with a plastic chassis, carefully slide the I/O daughter-board cable underneath the 
wireless-antenna cables to remove the I/O daughter-board cable.
Installing the I/O daughter-board cable
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following image indicates the location of the I/O daughter-board cable and provides a visual representation of the 
installation procedure.
Removing and installing Field Replaceable Units (FRUs) 69

Steps
1.Place the I/O daughter-board cable on the palm-rest and keyboard assembly.
NOTE:  For computers shipped with a plastic chassis, carefully slide the I/O daughter-board cable underneath the 
wireless-antenna cables to place the I/O daughter-board cable on the palm-rest and keyboard assembly.
2.Connect the I/O daughter-board cable to the connector on the I/O daughter-board and close the latch.
3.Connect the I/O daughter-board cable to the connector on the system board and close the latch.
Next steps
1.Install the fan.
2.Install the base cover .
3.Follow the procedure in After working inside your computer .
I/O daughter-board
Removing the I/O daughter-board
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
About this task
The following images indicate the location of the I/O daughter-board and provide a visual representation of the removal 
procedure.
70 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Remove the two screws (M2.5x5) that secure the left display hinge to the palm-rest and keyboard assembly.
2.Using a plastic scribe, lift the left display hinge away from the palm-rest and keyboard assembly to access the I/O daughter-
board.
3.Open the latch and disconnect the I/O daughter-board cable from the connector on the I/O daughter-board.
4.Open the latch and disconnect the fingerprint reader cable from the I/O daughter-board.
NOTE:  This step applies only to computers shipped with a fingerprint reader installed.
5.Remove the two screws (M2x3.5)  that secure the I/O daughter-board to the palm-rest and keyboard assembly.
6.Carefully slide and remove the I/O daughter-board at angle, from the palm-rest and keyboard assembly, to clear the ports 
from the port openings.
Installing the I/O daughter-board
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following images indicate the location of the I/O daughter-board and provide a visual representation of the installation 
procedure.
Removing and installing Field Replaceable Units (FRUs) 71

Steps
1.Align the ports on the I/O daughter-board to the port openings on the palm-rest and keyboard assembly.
2.Carefully slide and place the I/O daughter-board at an angle, on the palm-rest and keyboard assembly.
3.Align the screw holes on the I/O daughter-board to the screw holes on the palm-rest and keyboard assembly.
4.Replace the two screws (M2x3.5)  to secure the I/O daughter-board to the palm-rest and keyboard assembly.
5.Connect the fingerprint reader cable to the connector on the I/O daughter-board and close the latch.
NOTE:  This step applies only to computers shipped with a fingerprint reader installed.
6.Connect the I/O daughter-board cable to the connector on the I/O daughter-board and close the latch.
7.Close the left display hinge downwards to align the screw holes on the left display hinge to the screw holes on the I/O 
daughter-board and the palm-rest and keyboard assembly.
8.Replace the two screws (M2.5x5) to secure the left display hinge to the I/O daughter-baord and the palm-rest and keyboard 
assembly.
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
Power button
Removing the power button
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the I/O daughter-board .
72 Removing and installing Field Replaceable Units (FRUs)

About this task
The following image indicate s the location of the power button and provide s a visual representation of the removal procedure.
Steps
1.Remove the screw (M2x2) that secures the power button to the palm-rest and keyboard assembly.
2.Lift the power button off the palm-rest and keyboard assembly.
Installing the power button
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following image indicate s the location of the power button and provide s a visual representation of the installation 
procedure.
Removing and installing Field Replaceable Units (FRUs) 73

Steps
1.Place the power button in the slot on the palm-rest and keyboard assembly.
2.Align the screw hole on the power button to the screw hole on the palm-rest and keyboard assembly.
3.Replace the screw (M2x2) to secure the power button to the palm-rest and keyboard assembly.
Next steps
1.Install the I/O daughter-board .
2.Install the base cover .
3.Follow the procedure in After working inside your computer .
Power button with optional fingerprint reader
Removing the power button with optional fingerprint reader
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the I/O daughter-board .
About this task
NOTE:  This procedure applies only to computers shipped with a power button with optional fingerprint reader installed.
The following image indicates the location of the power button with optional fingerprint reader and provides a visual 
representation of the removal procedure.
74 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Peel back the tape that secures the optional fingerprint-reader flexible printed circuit to the palm-rest and keyboard 
assembly.
2.Remove the screw (M2x2) that secures the power button with optional fingerprint reader to the palm-rest and keyboard 
assembly.
3.Lift the power button with optional fingerprint reader off the slot on the palm-rest and keyboard assembly.
Installing the power button with optional fingerprint reader
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  This procedure applies only to computers shipped with a power button with optional fingerprint reader installed.
The following image indicates the location of the power button with optional fingerprint reader and provides a visual 
representation of the installation procedure.
Removing and installing Field Replaceable Units (FRUs) 75

Steps
1.Place the power button with optional fingerprint reader in the slot on the palm-rest and keyboard assembly.
2.Align the screw hole on the power button with optional fingerprint reader to the screw hole on the palm-rest and keyboard 
assembly.
3.Replace the screw (M2x2) to secure the power button with optional fingerprint reader to the palm-rest and keyboard 
assembly.
4.Adhere the tape to secure the optional fingerprint-reader flexible printed circuit to the palm-rest and keyboard assembly.
Next steps
1.Install the I/O daughter-board .
2.Install the base cover .
3.Follow the procedure in After working inside your computer .
Power-adapter port
Removing the power-adapter port
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
About this task
The following images indicate the location of the power-adapter port and provide a visual representation of the removal 
procedure.
76 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Remove the two screws (M2.5x5) that secure the right display hinge to the system board and the palm-rest and keyboard 
assembly.
2.Using a plastic scribe, lift the right display hinge away from the palm-rest and keyboard assembly to access the power-
adapter port.
3.Disconnect the power-adapter port cable from the connector on the system board.
4.Remove the power-adapter port from the slot on the palm-rest and keyboard assembly.
NOTE:  The power-adapter port is secured in place by the right display hinge. There is no screw, tape, or adhesives 
securing the power-adapter port to the palm rest. As a result, technicians should take note of whether the power-
adapter port has fallen out of the computer during subsequent removal procedures.
Installing the power-adapter port
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
The following images indicate the location of the power-adapter port and provide a visual representation of the installation 
procedure.
Removing and installing Field Replaceable Units (FRUs) 77

Steps
1.Align and place the power-adapter port in the slot on the palm-rest and keyboard assembly.
NOTE:  The power-adapter port is secured in place by the right display hinge. There is no screw, tape, or adhesives 
securing the power-adapter port to the palm rest. As a result, technicians should take note of whether the power-
adapter port has fallen out of the computer during subsequent removal procedures.
2.Connect the power-adapter port cable to the connector on the system board.
3.Close the right display hinge downwards to align the screw holes on the right display hinge to the screw holes on the system 
board and the palm-rest and keyboard assembly.
4.Replace the two screws (M2.5x5) to secure the right display hinge to the system board and the palm-rest and keyboard 
assembly.
Next steps
1.Install the base cover .
2.Follow the procedure in After working inside your computer .
Display assembly
Removing the display assembly
CAUTION:  The information in this section is intended for authorized service technicians only.
CAUTION:  The maximum operating angle for the display-panel hinge is 135 degrees.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the wireless card .
NOTE:  This procedure applies only to systems shipped with a plastic chassis.
78 Removing and installing Field Replaceable Units (FRUs)

About this task
The following images indicate the location of the display assembly and provide a visual representation of the removal procedure.
Removing and installing Field Replaceable Units (FRUs) 79

Steps
1.Disconnect the speaker cable from the connector on the system board.
NOTE:  This step applies only to computers shipped with a plastic chassis.
2.Disconnect the display cable from the connector on the system board.
3.Unroute the wireless-antenna cable from the routing guides on the palm-rest and keyboard assembly.
NOTE:  This step applies only to computers shipped with a plastic chassis.
4.Remove the four screws (M2.5x5) that secure the display hinges to the palm-rest and keyboard assembly.
5.Using a plastic scribe, lift the left and right hinges away from the palm-rest and keyboard assembly.
6.Lift the palm-rest and keyboard assembly at an angle to free it from the hinges and remove it from the display assembly.
NOTE:  For computers shipped with an aluminum chassis, the display assembly is a Hinge-Up Design (HUD) assembly 
and cannot be further disassembled once it is removed from the palm-rest and keyboard assembly. If any components in 
the display assembly are malfunctioning and are required to be replaced, replace the entire display assembly.
NOTE:  The power-adapter port is secured in place by the right display hinge. There is no screw, tape, or adhesives 
securing the power-adapter port to the palm-rest. As a result, technicians should take note of whether the power-
adapter port has fallen out of the computer during subsequent removal procedures.
Installing the display assembly
CAUTION:  The information in this section is intended for authorized service technicians only.
CAUTION:  The maximum operating angle for the display-panel hinge is 135 degrees.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
80 Removing and installing Field Replaceable Units (FRUs)

About this task
NOTE:  Ensure that the display hinges are opened to the maximum before replacing the display assembly on the palm-rest 
and keyboard assembly.
The following images indicate the location of the display assembly and provide a visual representation of the installation 
procedure.
Steps
1.Place the display assembly on a clean and flat surface.
Removing and installing Field Replaceable Units (FRUs) 81

2.Slide the palm-rest and keyboard assembly at an angle and place the palm-rest and keyboard assembly on the display 
assembly.
NOTE:  The power-adapter port is secured in place by the right display hinge. There is no screw, tape, or adhesives 
securing the power-adapter port to the palm-rest. As a result, technicians should take note of whether the power-
adapter port has fallen out of the computer during subsequent removal procedures.
3.Gently press down on the display hinges to align the screw holes on the display hinges with the screw holes on the palm-rest 
and keyboard assembly.
4.Replace the four screws (M2.5x5) to secure the display hinges to the palm-rest and keyboard assembly.
5.Route the wireless-antenna cables through the routing guides on the palm-rest and keyboard assembly.
NOTE:  This step applies only to computers shipped with a plastic chassis.
6.Route the speaker cable through the routing guides on the palm-rest and keyboard assembly.
NOTE:  This step applies only to computers shipped with a plastic chassis.
7.Connect the speaker cable to the connector on the system board.
NOTE:  This step applies only to computers shipped with a plastic chassis.
8.Connect the display cable to the connector on the system board.
Next steps
1.Install the wireless card .
NOTE:  This procedure applies only to systems shipped with a plastic chassis.
2.Install the base cover .
3.Follow the procedure in After working inside your computer .
82 Removing and installing Field Replaceable Units (FRUs)

Display hinge caps
Removing the display hinge caps
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the wireless card .
4.Remove the display assembly .
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following images indicate the location of the display hinge caps and provide a visual representation of the removal 
procedure.
Removing and installing Field Replaceable Units (FRUs) 83

Steps
1.Place the display assembly on a clean, flat surface and gently open the display hinges to at least 90 degrees.
84 Removing and installing Field Replaceable Units (FRUs)

2.Using a plastic scribe, pry open the left display-hinge cap from its right side and remove it from the left display hinge.
3.Using a plastic scribe, pry open the right display-hinge cap from its left side and remove it from the right display hinge.
Installing the display hinge caps
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following images indicate the location of the display hinge caps and provide a visual representation of the installation 
procedure.
Removing and installing Field Replaceable Units (FRUs) 85

Steps
1.Align the rib, inside the right display-hinge cap, to the left-outer side of the right display hinge.
2.Push the right display-hinge cap down on the right display hinge, until it clicks in place.
3.Align the rib, inside the left display-hinge cap, to the right-outer side of the left display hinge.
4.Push the left display-hinge cap down on the right display hinge, until it clicks in place.
Next steps
1.Install the display assembly .
2.Install the wireless card .
3.Install the base cover .
4.Follow the procedure in After working inside your computer .
86 Removing and installing Field Replaceable Units (FRUs)

Display bezel
Removing the display bezel
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the wireless card .
4.Remove the display assembly .
5.Remove the display hinge caps .
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following image indicates the location of the display bezel and provides a visual representation of the removal procedure.
Steps
1.Using a plastic scribe, pry open the display bezel from the outer edge of openings at the bottom side of the display assembly, 
near the display hinges.
Removing and installing Field Replaceable Units (FRUs) 87

2.Carefully pry open the outer edge of the bottom side of the display bezel.
3.Using the plastic scribe, pry open the outside edge of the left, right, and top sides of the display bezel.
4.Lift and remove the display bezel from the display assembly.
CAUTION:  DO NOT use a plastic scribe or any other objects to pry up the display bezel in the manner shown 
below, as the pressure applied on the display panel by the scribe may damage the display panel.
88 Removing and installing Field Replaceable Units (FRUs)

Installing the display bezel
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following image indicates the location of the display bezel and provides a visual representation of the installation procedure.
Removing and installing Field Replaceable Units (FRUs) 89

Steps
1.Place the display assembly on a clean and flat surface.
2.Align and place the display bezel on the display assembly.
3.Starting from the top corner, press on the display bezel and work around the entire bezel until it clicks onto the display 
assembly.
Next steps
1.Install the display hinge caps .
2.Install the display assembly .
3.Install the wireless card .
4.Install the base cover .
5.Follow the procedure in After working inside your computer .
Display hinges
Removing the display hinges
CAUTION:  The information in this section is intended for authorized service technicians only.
90 Removing and installing Field Replaceable Units (FRUs)

Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the wireless card .
4.Remove the display assembly .
5.Remove the display hinge caps .
6.Remove the display bezel .
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following images indicate the location of the display hinges and provide a visual representation of the removal procedure.
Steps
1.Remove the three screws (M2.5x3.8) that secure the left display hinge to the display panel and back cover.
2.Lift and remove the left display hinge from the display assembly.
3.Remove the three screws (M2.5x3.8) that secure the right display hinge to the display panel and back cover.
4.Lift and remove the right display hinge from the display assembly.
Removing and installing Field Replaceable Units (FRUs) 91

Installing the display hinges
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following images indicate the location of the display hinges and provide a visual representation of the installation procedure.
Steps
1.Align and place the left display hinge on the display assembly.
2.Replace the three screws (M2.5x3.8) to secure the left display hinge to the display panel and back cover.
3.Align and place the right display hinge on the display assembly.
4.Replace the three screws (M2.5x3.8) to secure the right display hinge to the display panel and back cover.
92 Removing and installing Field Replaceable Units (FRUs)

Next steps
1.Install the display bezel .
2.Install the display hinge caps .
3.Install the display assembly .
4.Install the wireless card .
5.Install the base cover .
6.Follow the procedure in After working inside your computer .
Display panel
Removing the display panel
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the wireless card .
4.Remove the display assembly .
5.Remove the display hinge caps .
6.Remove the display bezel .
7.Remove the display hinges .
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following images indicate the location of the display panel and provide a visual representation of the removal procedure.
Removing and installing Field Replaceable Units (FRUs) 93

94 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Using a plastic scribe, pry the display panel from the bottom-right corner.
2.Using your hands, gently pry the display panel from the tabs along the right side of the display back-cover and antenna 
assembly.
Removing and installing Field Replaceable Units (FRUs) 95

3.Repeat step 1 and 2 for the left side of the display panel assembly.
96 Removing and installing Field Replaceable Units (FRUs)

4.Lift the bottom side of the display panel and slide it downward to release the display brackets from the slots at the top side 
of the display back-cover.
5.Gently flip the display panel assembly forward, peel back the tape that secures the display cable to the connector on the rear 
of the display panel.
NOTE:  Ensure that the panel has a clean and smooth surface to rest on, to prevent damage.
6.Disconnect the display cable from the connector on the display panel and remove the display panel.
NOTE:  The display panel is assembled with the display brackets as a single service part. DO NOT pull the two pieces of 
elastic tape and separate the brackets from the panel.
Removing and installing Field Replaceable Units (FRUs) 97

Installing the display panel
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following images indicate the location of the display panel and provide a visual representation of the installation procedure.
98 Removing and installing Field Replaceable Units (FRUs)

Removing and installing Field Replaceable Units (FRUs) 99

Steps
1.Place the display panel and display assembly on a clean and flat surface.
2.Connect the display cable to the connector on the display panel and close the latch.
3.Adhere the tape to secure the display cable to the connector on the display panel.
4.Turn the display panel over and place the display panel on the display back-cover.
5.Lift the bottom side of the display panel and slide the top side into the slots at the top of the display back-cover.
6.Gently push down on the edges of the display panel until it clicks into the tabs on the display back-cover.
Next steps
1.Install the display hinges .
2.Install the display bezel .
3.Install the display hinge caps .
4.Install the display assembly .
5.Install the wireless card .
6.Install the base cover .
7.Follow the procedure in After working inside your computer .
100 Removing and installing Field Replaceable Units (FRUs)

Camera
Removing the camera
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the wireless card .
4.Remove the display assembly .
5.Remove the display hinge caps .
6.Remove the display bezel .
7.Remove the display hinges .
8.Remove the display panel .
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following images indicate the location of the camera module and provide a visual representation of the removal procedure.
Steps
1.Disconnect the display cable from the connector on the camera module.
2.Using a plastic scribe, gently pry the camera off the display back-cover and antenna assembly.
3.Remove the camera module from the display assembly.
Installing the camera
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
Removing and installing Field Replaceable Units (FRUs) 101

About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following images indicate the location of the camera module and provide a visual representation of the installation 
procedure.
Steps
1.Using the alignment post, adhere the camera module on the display back-cover and antenna assembly.
2.Connect the display cable to the connector on the camera module.
Next steps
1.Install the display panel .
2.Install the display hinges .
3.Install the display bezel .
4.Install the display hinge caps .
5.Install the display assembly .
6.Install the wireless card .
7.Install the base cover .
8.Follow the procedure in After working inside your computer .
Display cable
Removing the display cable
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the wireless card .
4.Remove the display assembly .
5.Remove the display hinge caps .
6.Remove the display bezel .
7.Remove the display hinges .
102 Removing and installing Field Replaceable Units (FRUs)

8.Remove the display panel .
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following image indicates the location of the display cable and provides a visual representation of the removal procedure.
Steps
1.Disconnect the display eDP cable from the connector on the camera module.
2.Carefully peel back and remove the display eDP cable from the display back-cover and antenna assembly.
Installing the display cable
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following image indicates the location of the display cable and provides a visual representation of the installation procedure.
Removing and installing Field Replaceable Units (FRUs) 103

Steps
1.Adhere the display eDP cable to the display back-cover and antenna assembly.
2.Connect the display eDP cable to the connector on the camera module.
Next steps
1.Install the display panel .
2.Install the display hinges .
3.Install the display bezel .
4.Install the display hinge caps .
5.Install the display assembly .
6.Install the wireless card .
7.Install the base cover .
8.Follow the procedure in After working inside your computer .
Display back-cover and antenna assembly
Removing the display back-cover and antenna assembly
CAUTION:  The information in this section is intended for authorized service technicians only.
104 Removing and installing Field Replaceable Units (FRUs)

Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the wireless card .
4.Remove the display assembly .
5.Remove the display hinge caps .
6.Remove the display bezel .
7.Remove the display hinges .
8.Remove the display panel .
9.Remove the camera .
10.Remove the display cable .
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
NOTE:  The display back-cover and antenna assembly cannot be further disassembled once all the pre-removal parts 
procedures are completed. If the wireless antennas are malfunctioning and are required to be replaced, replace the entire 
display back-cover and antenna assembly.
The image below shows the display back-cover and antenna assembly after the pre-removal parts procedures have been 
performed for any display back-cover and antenna assembly replacement.
Steps
After performing the pre-requisites, you are left with the display back-cover and antenna assembly.
Installing the display back-cover and antenna assembly
CAUTION:  The information in this section is intended for authorized service technicians only.
Removing and installing Field Replaceable Units (FRUs) 105

Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  This procedure applies only to computers with a plastic chassis.
The following image indicates the location of the display back-cover and antenna assembly and provides a visual representation 
of the installation procedure.
Steps
Place the display back-cover and antenna assembly on a flat surface and perform the post-requisites to install the display 
back-cover and antenna assembly.
Next steps
1.Install the display cable .
2.Install the camera .
3.Install the display panel .
4.Install the display hinges .
5.Install the display bezel .
6.Install the display hinge caps .
7.Install the display assembly .
8.Install the wireless card .
9.Install the base cover .
10.Follow the procedure in After working inside your computer .
106 Removing and installing Field Replaceable Units (FRUs)

System board
Removing the system board
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the memory module .
4.Remove the M.2 2230 solid-state drive  or the M.2 2280 solid-state drive , whichever applicable.
5.Remove the wireless card .
6.Remove the fan.
7.Remove the heat sink - integrated  or the heat sink - discrete , whichever applicable.
NOTE:  When removing the system board to replace/access other parts, the system board can be removed with the 
heat sink attached to it in order to simplify the procedure and preserve the thermal bond between the system board and 
heat sink.
About this task
The following image indicates the connectors on your system board.
1.Fan cable connector
2.I/O daughter-board FFC connector
3.eDP connector
4.Power-adapter port connector
5.Memory module connector
6.Keyboard FFC connector
7.Keyboard-backlight FPC connector
8.Battery connector
9.Touchpad FFC connector
10.Wireless card connector
11.Speaker cable connector
Removing and installing Field Replaceable Units (FRUs) 107

12.M.2 solid-state drive connector
The following images indicate the location of the system board and provide a visual representation of the removal procedure.
Steps
1.Remove the two screws (M2.5x5) that secure the right display hinge to the system board and the palm-rest and keyboard 
assembly.
2.Using a plastic scribe, lift the right display hinge away from the palm-rest and keyboard assembly.
3.Disconnect the following cables from the system board:
a.I/O daughter-board FFC
b.eDP cable
c.Power-adapter port cable
d.Keyboard FFC
e.Keyboard-backlight FPC
NOTE:  This step applies only to computers shipped with a keyboard backlight installed.
108 Removing and installing Field Replaceable Units (FRUs)

f.Battery cable
g.Touchpad FFC
h.Speaker cable
4.Remove the screw (M2x2) and the four screws (M2x3.5) that secure the system board to the palm-rest and keyboard 
assembly.
NOTE:  DO NOT remove the USB Type-C bracket from the system board, for individual replacement. The system board 
and the USB Type-C bracket are replaced as an assembly part.
5.Carefully lift and remove the system board from the palm-rest and keyboard assembly.
Installing the system board
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  When installing the system board to replace/access other parts, the system board can be installed with the heat 
sink attached to it in order to simplify the procedure and preserve the thermal bond between the system board and heat 
sink.
The following image indicates the connectors on your system board.
Removing and installing Field Replaceable Units (FRUs) 109

1.Fan cable connector
2.I/O daughter-board FFC connector
3.eDP connector
4.Power-adapter port connector
5.Memory module connector
6.Keyboard FFC connector
7.Keyboard-backlight FPC connector
8.Battery connector
9.Touchpad FFC connector
10.Wireless card connector
11.Speaker cable connector
12.M.2 solid-state drive connector
The following images indicate the location of the system board and provide a visual representation of the installation procedure.
110 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Align the ports on the system board to the port openings on the palm-rest and keyboard assembly.
2.At an angle, carefully slide and place the system board on the palm-rest and keyboard assembly.
3.Align the screw holes on the system board with the screw holes on the palm-rest and keyboard assembly.
4.Replace the screw (M2x2) and the four screws (M2x3.5) to secure the system board to the palm-rest and keyboard 
assembly.
5.Connect the following cables to the system board:
a.I/O daughter-board FFC
b.eDP cable
c.Power-adapter port cable
d.Keyboard FFC
e.Keyboard-backlight FPC
NOTE:  This step applies only to computers shipped with a keyboard backlight installed.
Removing and installing Field Replaceable Units (FRUs) 111

f.Battery cable
g.Touchpad FFC
h.Speaker cable
6.Close the right display hinge downwards to align the screw holes on the right display hinge to the screw holes on the system 
board and the palm-rest and keyboard assembly.
7.Replace the two screws (M2.5x5) to secure the right display hinge to the system board and the palm-rest and keyboard 
assembly.
Next steps
1.Install the heat sink - integrated  or the heat sink - discrete , whichever applicable.
2.Install the fan.
3.Install the wireless card .
4.Install the M.2 2230 solid-state drive  or the M.2 2280 solid-state drive , whichever applicable.
5.Install the memory module .
6.Install the base cover .
7.Follow the procedure in After working inside your computer .
Palm-rest and keyboard assembly
Removing the palm-rest and keyboard assembly
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
1.Follow the procedure in Before working inside your computer .
2.Remove the base cover .
3.Remove the memory module .
4.Remove the M.2 2230 solid-state drive  or the M.2 2280 solid-state drive , whichever applicable.
5.Remove the wireless card .
6.Remove the speakers .
7.Remove the fan.
8.Remove the heat sink - integrated  or the heat sink - discrete , whichever applicable.
9.Remove the 3-cell battery  or the 4-cell battery , whichever applicable.
10.Remove the touchpad .
11.Remove the I/O daughter-board cable .
12.Remove the I/O daughter-board .
13.Remove the power button  or the power button with optional fingerprint reader , whichever applicable.
14.Remove the power-adapter port .
15.Remove the display assembly .
16.Remove the system board .
NOTE:  When removing the system board to replace/access other parts, the system board can be removed with the 
heat sink attached to it in order to simplify the procedure and preserve the thermal bond between the system board and 
heat sink.
About this task
NOTE:  The palm-rest and keyboard assembly cannot be further disassembled once all the pre-removal parts procedures 
are completed. If the keyboard is malfunctioning and is required to be replaced, replace the entire palm-rest and keyboard 
assembly.
The image below shows the palm-rest and keyboard assembly after the pre-removal parts procedures have been performed for 
any palm-rest and keyboard assembly replacement.
112 Removing and installing Field Replaceable Units (FRUs)

Steps
1.For computers shipped with an aluminum chassis and a 4-cell battery installed, remove the two screws (M2x2) that secure 
the left RF metal wall bracket to the palm-rest and keyboard assembly.
2.For computers shipped with an aluminum chassis, remove the right and left RF metal wall bracket from the palm-rest and 
keyboard assembly.
NOTE:  The left and right RF metal wall brackets MUST be transferred to the replacement palm-rest and keyboard 
assembly.
Removing and installing Field Replaceable Units (FRUs) 113

3.After performing the pre-requisites, you are left with the palm-rest and keyboard assembly.
Installing the palm-rest and keyboard assembly
CAUTION:  The information in this section is intended for authorized service technicians only.
Prerequisites
If you are replacing a component, remove the existing component before performing the installation procedure.
About this task
NOTE:  The system board can be installed with the heat sink attached to it in order to simplify the procedure and preserve 
the thermal bond between the system board and heat sink.
The following image indicates the location of the palm-rest and keyboard assembly and provides a visual representation of the 
installation procedure.
114 Removing and installing Field Replaceable Units (FRUs)

Steps
1.Place the palm-rest and keyboard assembly on a flat surface and perform the post-requisites to install the palm-rest and 
keyboard assembly.
2.For computers shipped with an aluminum chassis, place the right and left RF metal wall bracket in the slot on the palm-rest 
and keyboard assembly.
NOTE:  The left and right RF metal wall brackets MUST be transferred to the replacement palm-rest and keyboard 
assembly.
3.For computers shipped with an aluminum chassis and a 4-cell battery installed, replace the two screws (M2x2) to secure the 
left RF metal wall bracket to the palm-rest and keyboard assembly.
Removing and installing Field Replaceable Units (FRUs) 115

Next steps
1.Install the system board .
2.Install the display assembly .
3.Install the power-adapter port .
4.Install the power button  or the power button with optional fingerprint reader , whichever applicable.
5.Install the I/O daughter-board .
6.Install the I/O daughter-board cable .
7.Install the touchpad .
8.Install the 3-cell battery  or the 4-cell battery , whichever applicable.
9.Install the heat sink - integrated  or the heat sink - discrete , whichever applicable.
10.Install the fan.
11.Install the speakers .
12.Install the wireless card .
13.Install the M.2 2230 solid-state drive  or the M.2 2280 solid-state drive , whichever applicable.
14.Install the memory module .
15.Install the base cover .
16.Follow the procedure in After working inside your computer .
116 Removing and installing Field Replaceable Units (FRUs)

Software
This chapter details the supported operating systems along with instructions on how to install the drivers.
Operating system
Your Vostro 15 3530  supports the following operating systems:
●Windows 11 Pro
●Windows 11 Pro National Academic
●Windows 11 Home
●Windows 11 Pro Downgrade (Windows 10 image factory installed)
●Ubuntu 22.04 LTS, 64-bit
Drivers and downloads
When troubleshooting, downloading, or installing drivers, it is recommended that you read the Dell Knowledge Base article 
Drivers and Downloads FAQs 000123347 .7
Software 117

BIOS Setup
CAUTION:  Unless you are an expert computer user, do not change the settings in the BIOS Setup. Certain 
changes can make your computer work incorrectly.
NOTE:  Depending on the computer and its installed devices, the items that are listed in this section may or may not be 
displayed.
NOTE:  Before you change the settings in BIOS Setup, it is recommended that you note down the original settings for 
future reference.
Use BIOS Setup for the following purposes:
●Get information about the hardware installed in your computer, such as the amount of RAM and the size of the storage 
device.
●Change the system configuration information.
●Set or change a user-selectable option, such as the user password, type of hard drive installed, and enabling or disabling 
base devices.
Entering BIOS setup program
About this task
Turn on (or restart) your computer and press F2 immediately.
Navigation keys
NOTE:  For most of the System Setup options, changes that you make are recorded but do not take effect until you restart 
the computer.
Table 30. Navigation keys 
Keys Navigation
Up arrow Moves to the previous field.
Down arrow Moves to the next field.
Enter Selects a value in the selected field (if applicable) or follows 
the link in the field.
Spacebar Expands or collapses a drop-down list, if applicable.
Tab Moves to the next focus area.
NOTE:  For the standard graphical user interface only.
Esc Moves to the previous page until you view the main screen. 
Pressing Esc in the main screen displays a message that 
prompts you to save any unsaved changes and restart the 
computer.
One time boot menu
To enter one time boot menu , turn on your computer, and then press F12 immediately.8
118 BIOS Setup

NOTE:  It is recommended to shutdown the computer if it is on.
The one-time boot menu displays the devices that you can boot from including the diagnostic option. The boot menu options 
are:
●Windows Boot Manager
●UEFI HTTPs Boot
●UEFI M.2 solid-state drive Boot
●ONBOARD NIC (IPV4)
●ONBOARD NIC (IPV6)
The boot sequence screen also displays the option to access the System Setup screen.
System setup options
NOTE:  Depending on your computer and its installed devices, the items that are listed in this section may or may not 
appear.
Table 31. System setup options — System information menu 
Overview
Vostro 15 3530
BIOS Version Displays the BIOS version number of the computer.
Service Tag Displays the Service Tag of the computer.
Asset Tag Displays the Asset Tag of the computer.
Manufacture Date Displays the manufacture date of the computer.
Ownership Date Displays the ownership date of the computer.
Express Service Code Displays the express service code of the computer.
Ownership Tag Displays the Ownership Tag of the computer.
Signed Firmware Update Displays whether the Signed Firmware Update is enabled on your computer.
By default, the option is enabled.
Battery
Primary Displays that battery is primary.
Battery Level Displays the battery level of the computer.
Battery State Displays the battery state of the computer.
Health Displays the battery health of the computer.
AC Adapter Displays whether an AC adapter is connected or not. If connected, displays the 
AC adapter type.
Battery Life Type Displays the battery life type of the computer.
Processor
Processor Type Displays the processor type.
Maximum Clock Speed Displays the maximum processor clock speed.
Minimum Clock Speed Displays the minimum processor clock speed.
Current Clock Speed Displays the current processor clock speed.
Core Count Displays the number of cores on the processor.
Processor ID Displays the processor identification code.
Processor L2 Cache Displays the processor L2 Cache size.
Processor L3 Cache Displays the processor L3 Cache size.
BIOS Setup 119

Table 31. System setup options — System information menu (continued)
Overview
Microcode Version Displays the microcode version.
Intel Hyper-Threading Capable Displays whether the processor is Hyper-Threading (HT) capable or not.
64-Bit Technology Displays whether 64-bit technology is used or not.
Memory
Memory Installed Displays the total computer memory installed.
Memory Available Displays the total computer memory available.
Memory Speed Displays the memory speed.
Memory Channel Mode Displays single or dual channel mode.
Memory Technology Displays the technology that is used for the memory.
DIMM A Size Displays the total DIMM A computer memory available, with memory type.
DIMM B Size Displays the total DIMM B computer memory available, with memory type.
Devices
Panel Type Displays the panel type of the computer.
Video Controller Displays the video controller information of the computer.
Video Memory Displays the video memory information of the computer.
Wi-Fi Device Displays the wireless device information of the computer.
Native Resolution Displays the native resolution of the computer.
Video BIOS Version Displays the video BIOS version of the computer.
Audio Controller Displays the audio controller information of the computer.
Bluetooth Device Displays the Bluetooth device information of the computer.
LOM MAC Address Displays the MAC address of the computer.
Table 32. System setup options — Boot Configuration menu 
Boot Configuration
Boot Sequence
Boot mode Displays the boot mode of the computer.
Boot Sequence Specifies the order that the BIOS searches the list of devices to find an 
operating system to boot.
By default, the Windows Boot Manager  option is selected.
By default, the UEFI HTTPs Boot (MAC:04BF1B414407)  option is selected.
By default, the UEFI RST CL4-3D512-Q11 NVMe SSSTC 512GB 
VFN39DH002780885  option is selected.
By default, the ONBOARD NIC (IPV4)  option is selected.
By default, the ONBOARD NIC (IPV6)  option is selected.
Secure Boot
Enable Secure Boot Displays the boot mode of the computer.
By default, the option is disabled.
Enable Microsoft UEFI CA When disabled, the UEFI CA is removed from the BIOS UEFI Secure Boot 
database.
120 BIOS Setup

Table 32. System setup options — Boot Configuration menu (continued)
Boot Configuration
NOTE:  When disabled, the Microsoft UEFI CA could render your computer 
unable to boot, system graphics may not function, some devices may not 
function properly, and the computer could become unrecoverable.
By default, the option is enabled.
Secure Boot Mode Enables or disables the Secure Boot operation mode.
By default, the Deployed Mode  option is selected.
Expert Key Management
Enable Custom Mode Enables or disables the keys in the PK, KEK, db, and dbx security key databases 
to be modified.
By default, the option is disabled.
Custom Mode Key Management Allows the user to select the custom values for expert key management.
By default, PK option is selected.
Table 33. System setup options — Integrated Devices menu 
Integrated Devices
Date/Time
Date Sets the computer date in MM/DD/YYYY format. Changes to the date take 
effect immediately.
Time Sets the computer time in HH:MM:SS 24-hour format. You can switch 
between 12-hour and 24-hour clock. Changes to the time take effect 
immediately.
Camera
Enable Camera Enables or disables the camera.
By default, the option is enabled.
Audio Enables or disables the integrated audio controller.
By default, all the options are enabled.
Enable Microphone Enables or disables the microphone.
By default, the option is enabled.
Enable Internal Speaker Enables or disables the internal speaker.
By default, the option is enabled.
USB Configuration
Enable USB Boot Support Enables or disables the USB boot support.
When enabled, bootable USB mass storage devices (such as HDD, flash drive, 
CD/DVD) can boot through the boot sequence or boot menu. USB ports are 
also functional in an OS environment.
When disabled, bootable USB mass storage devices are prevented from 
booting through the boot sequence or boot menu but the USB ports are 
functional in an OS environment.
By default, the option is enabled.
BIOS Setup 121

Table 34. System setup options — Storage menu 
Storage
SATA/NVMe Operation Allows the user to set the operating mode of the integrated storage device 
controller.
By default, RAID On  option is selected.
Storage interface
Port Enablement Enables or disables the onboard drives.
By default, the SATA-0  option is enabled.
By default, the M.2 PCIe SSD  option is enabled.
Drive Information
SATA-0
Type Displays the SATA-0 type information of the computer.
Device Displays the SATA-0 device information of the computer.
M.2 PCIe SSD
Type Displays the M.2 PCIe SSD type information of the computer.
Device Displays the M.2 PCIe SSD device information of the computer.
Table 35. System setup options — Display menu 
Display
Display Brightness
Brightness on battery power Allows the user to set the screen brightness level when the computer is 
running on battery power.
By default, the Brightness on battery power  level is set to 70.
Brightness on AC power Allows the user to set the screen brightness level when the computer is 
running on AC power.
By default, the Brightness on AC power  level is set to 100.
Table 36. System setup options — Security menu 
Security
TPM 2.0 Security 
Clear When enabled, the Clear option clears information stored in the TPM after 
exiting the system's BIOS. This option returns to disabled state when the 
system restarts.
By default, the option is disabled.
PPI Bypass for Clear Commands Enables or disables the TPM Physical Presence Interface (PPI).
When enabled, this setting allows the operating system to skip BIOS PPI user 
prompts when issuing the Clear command. Changes to this setting take effect 
immediately.
By default, the option is disabled.
Data Wipe on Next Boot 
Start Data Wipe Enables or disables the Data Wipe feature.
When enabled, the BIOS will queue up a data wipe cycle for storage device(s), 
connected to the computer, on next reboot.
WARNING:  Secure Data Wipe operation deletes information in a way 
that it cannot be reconstructed.
122 BIOS Setup

Table 36. System setup options — Security menu (continued)
Security
By default, the option is disabled.
Table 37. System setup options — Passwords menu 
Passwords
Admin Password Enables the user to set, change, or delete the administrator (admin) password. 
The administrator password enables several security features.
System Password Enables the user to set, change, or delete the system password. The system 
password prevents the computer from booting to an operating system without 
entering the correct password.
M.2 PCIe SSD-0 Enables the user to set, change, or delete the M.2 PCIe SSD-0 password.
Table 38. System setup options — Update, Recovery menu 
Update, Recovery
SupportAssist OS Recovery Enables or disables the boot flow for SupportAssist OS Recovery tool, in the 
event of certain system error.
By default, the option is enabled.
BIOSConnect Enables or disables cloud Service OS recovery if the main OS fails to boot 
within the number of failures equal or greater than the value specified by 
Dell Auto OS Recovery Threshold, and local Service does not boot, or is not 
installed.
By default, the option is enabled.
Dell Auto OS Recovery Threshold Controls the automatic boot flow for SupportAssist System Resolution Console 
and for Dell operating system Recovery tool.
By default, the Dell Auto OS Recovery Threshold  value is set to 2.
Table 39. System setup options — System Management menu 
System Management
Service Tag Displays the Service Tag of the computer.
Asset Tag Creates a system Asset Tag that can be used by an IT administrator to 
uniquely identify a particular system. Once set in BIOS, the Asset Tag cannot 
be changed.
Diagnostics 
OS Agent Requests Enables or disables the Dell OS Agent to schedule onboard diagnostics on a 
subsequent boot. Enabling this option helps assist in prevention and resolution 
of hardware related issues.
By default, the option is enabled.
Table 40. System setup options — Keyboard menu 
Keyboard
Keyboard illumination Allows the user to select the keyboard illumination brightness.
By default, the Dim option is selected. This option enables the keyboard 
illumination feature at 50% brightness level.
Keyboard Backlight Timeout on AC Allows the user to set the keyboard backlight timeout duration when the 
computer is running on AC power. The keyboard backlight timeout value is only 
in effect when the backlight is enabled.
By default, the Keyboard Backlight Timeout on AC  value is set to 1 minute.
BIOS Setup 123

Table 40. System setup options — Keyboard menu (continued)
Keyboard
Keyboard Backlight Timeout on 
battery Allows the user to set the keyboard backlight timeout duration when the 
computer is running on battery power. The keyboard backlight timeout value 
is only in effect when the backlight is enabled.
By default, the Keyboard Backlight Timeout on battery  value is set to 1 
minute.
Table 41. System setup options — Pre-boot Behavior menu 
Pre-boot Behavior
Adapter Warnings
Enable Adapter Warnings Enables or disables the computer to display adapter warning messages when 
adapters with too little power capacity are detected.
By default, the option is enabled.
Warning and Errors Enables or disables the action to be taken when a warning or error is 
encountered.
By default, the Prompt on Warnings and Errors  option is enabled. It stops, 
prompts, and waits for user input when warnings or errors are detected.
NOTE:  Errors deemed critical to the operation of the computer hardware 
will always halt the computer.
Table 42. System setup options — System Logs menu 
System Logs
BIOS Event Log
Clear Bios Event Log Displays BIOS events.
By default, the Keep Log  option is selected.
Thermal Event Log
Clear Thermal Event Log Displays Thermal events.
By default, the Keep Log  option is selected.
Power Event Log
Clear Power Event Log Displays power events.
By default, the Keep Log  option is selected.
Updating the BIOS
Updating the BIOS in Windows
About this task
CAUTION:  If BitLocker is not suspended before updating the BIOS, the next time you reboot the computer it 
will not recognize the BitLocker key. You will then be prompted to enter the recovery key to progress, and the 
computer will ask for this on each reboot. If the recovery key is not known this can result in data loss or an 
unnecessary operating system reinstall. For more information about this subject, search in the Knowledge Base 
Resource at www.dell.com/support .
124 BIOS Setup

Steps
1.Go to www.dell.com/support .
2.Click Product support . In the Search support  box, enter the Service Tag of your computer, and then click Search .
NOTE:  If you do not have the Service Tag, use the SupportAssist feature to automatically identify your computer. You 
can also use the product ID or manually browse for your computer model.
3.Click Drivers & Downloads . Expand Find drivers .
4.Select the operating system installed on your computer.
5.In the Category  drop-down list, select BIOS .
6.Select the latest version of BIOS, and click Download  to download the BIOS file for your computer.
7.After the download is complete, browse the folder where you saved the BIOS update file.
8.Double-click the BIOS update file icon and follow the on-screen instructions.
For more information, search in the Knowledge Base Resource at www.dell.com/support .
Updating the BIOS in Linux and Ubuntu
To update the system BIOS on a computer that is installed with Linux or Ubuntu, see the knowledge base article 000131486  at 
www.dell.com/support .
Updating the BIOS using the USB drive in Windows
About this task
CAUTION:  If BitLocker is not suspended before updating the BIOS, the next time you reboot the computer it 
will not recognize the BitLocker key. You will then be prompted to enter the recovery key to progress, and the 
computer will ask for this on each reboot. If the recovery key is not known this can result in data loss or an 
unnecessary operating system reinstall. For more information about this subject, search in the Knowledge Base 
Resource at www.dell.com/support .
Steps
1.Follow the procedure from step 1 to step 6 in Updating the BIOS in Windows  to download the latest BIOS setup program file.
2.Create a bootable USB drive. For more information, search in the Knowledge Base Resource at www.dell.com/support .
3.Copy the BIOS setup program file to the bootable USB drive.
4.Connect the bootable USB drive to the computer that needs the BIOS update.
5.Restart the computer and press F12 .
6.Select the USB drive from the One Time Boot Menu .
7.Type the BIOS setup program filename and press Enter .
The BIOS Update Utility  appears.
8.Follow the on-screen instructions to complete the BIOS update.
Updating the BIOS from the F12 One-Time boot menu
Update your computer BIOS using the BIOS update.exe file that is copied to a FAT32 USB drive and booting from the F12 
One-Time boot menu.
About this task
CAUTION:  If BitLocker is not suspended before updating the BIOS, the next time you reboot the computer it 
will not recognize the BitLocker key. You will then be prompted to enter the recovery key to progress, and the 
computer will ask for this on each reboot. If the recovery key is not known this can result in data loss or an 
unnecessary operating system reinstall. For more information about this subject, search in the Knowledge Base 
Resource at www.dell.com/support .
BIOS Setup 125

BIOS Update
You can run the BIOS update file from Windows using a bootable USB drive or you can also update the BIOS from the F12 
One-Time boot menu on the computer.
Most of the Dell computers built after 2012 have this capability, and you can confirm by booting your computer to the F12 
One-Time Boot Menu to see if BIOS FLASH UPDATE is listed as a boot option for your computer. If the option is listed, then the 
BIOS supports this BIOS update option.
NOTE:  Only computers with the BIOS Flash Update option in the F12 One-Time boot menu can use this function.
Updating from the One-Time boot menu
To update your BIOS from the F12 One-Time boot menu, you need the following:
●USB drive formatted to the FAT32 file system (key does not have to be bootable)
●BIOS executable file that you downloaded from the Dell Support website and copied to the root of the USB drive
●AC power adapter that is connected to the computer
●Functional computer battery to flash the BIOS
Perform the following steps to perform the BIOS update flash process from the F12 menu:
CAUTION:  Do not turn off the computer during the BIOS update process. The computer may not boot if you turn 
off your computer.
Steps
1.From a turn off state, insert the USB drive where you copied the flash into a USB port of the computer.
2.Turn on the computer and press F12 to access the One-Time Boot Menu, select BIOS Update using the mouse or arrow keys 
then press Enter.
The flash BIOS menu is displayed.
3.Click Flash from file .
4.Select an external USB device.
5.Select the file and double-click the flash target file, and then click Submit .
6.Click Update BIOS . The computer restarts to flash the BIOS.
7.The computer will restart after the BIOS update is completed.
System and setup password
Table 43. System and setup password 
Password type Description
System password Password that you must enter to log in to your system.
Setup password Password that you must enter to access and make changes to 
the BIOS settings of your computer.
You can create a system password and a setup password to secure your computer.
CAUTION:  The password features provide a basic level of security for the data on your computer.
CAUTION:  Anyone can access the data that is stored on your computer, when not locked and left unattended.
NOTE:  System and setup password feature is disabled.
Assigning a System Setup password
Prerequisites
You can assign a new System or Admin Password only when the status is in Not Set .
126 BIOS Setup

About this task
To enter BIOS System Setup, press F2 immediately after a power-on or reboot.
Steps
1.In the System BIOS  or System Setup  screen, select Security  and press Enter.
The Security  screen is visible.
2.Select System/Admin Password  and create a password in the Enter the new password  field.
Use the following guidelines to assign the system password:
●A password can have up to 32 characters.
●At least one special character: "( ! " # $ % & ' * + , - . / : ; < = > ? @ [ \ ] ^ _ ` { | } )"
●Numbers 0 to 9.
●Upper case letters from A to Z.
●Lower case letters from a to z.
3.Type the system password that you entered earlier in the Confirm new password  field and click OK.
4.Press Esc and save the changes as prompted by the message.
5.Press Y to save the changes.
The computer restarts.
Deleting or changing an existing system setup password
Prerequisites
Ensure that the Password Status  is Unlocked (in the System Setup) before attempting to delete or change the existing 
System and/or Setup password. You cannot delete or change an existing System or Setup password, if the Password Status  is 
Locked.
About this task
To enter the System Setup, press F2 immediately after a power-on or reboot.
Steps
1.In the System BIOS  or System Setup  screen, select System Security  and press Enter.
The System Security  screen is displayed.
2.In the System Security  screen, verify that the Password Status is Unlocked .
3.Select System Password , update, or delete the existing system password, and press Enter or Tab.
4.Select Setup Password , update, or delete the existing setup password, and press Enter or Tab.
NOTE:  If you change the System and/or Setup password, reenter the new password when prompted. If you delete the 
System and/or Setup password, confirm the deletion when prompted.
5.Press Esc. A message prompts you to save the changes.
6.Press Y to save the changes and exit from System Setup.
The computer restarts.
Clearing BIOS (System Setup) and System passwords
About this task
To clear the system or BIOS passwords, contact Dell technical support as described at www.dell.com/contactdell .
NOTE:  For information about how to reset Windows or application passwords, see the documentation accompanying 
Windows or your application.
BIOS Setup 127

Troubleshooting
Handling swollen rechargeable Li-ion batteries
Like most laptops, Dell laptops use Lithium-ion batteries. One type of Lithium-ion battery is the rechargeable Li-ion battery. 
Rechargeable Li-ion batteries have increased in popularity in recent years and have become standard in the electronics industry 
due to customer preferences for a slim form factor (especially with newer ultra-thin laptops) and long battery life. Inherent to 
rechargeable Li-ion battery technology is the potential for swelling of the battery cells.
A swollen battery may impact the performance of the laptop. To prevent possible further damage to the device enclosure or 
internal components leading to malfunction, discontinue the use of the laptop and discharge it by disconnecting the AC adapter 
and letting the battery drain.
Swollen batteries should not be used and should be replaced and disposed of properly. We recommend contacting Dell product 
support for options to replace a swollen battery under the terms of the applicable warranty or service contract, including options 
for replacement by a Dell authorized service technician.
The guidelines for handling and replacing rechargeable Li-ion batteries are as follows:
●Exercise caution when handling rechargeable Li-ion batteries.
●Discharge the battery before removing it from the system. To discharge the battery, unplug the AC adapter from the system 
and operate the system only on battery power. When the system will no longer turn on when the power button is pressed, 
the battery is fully discharged.
●Do not crush, drop, mutilate, or penetrate the battery with foreign objects.
●Do not expose the battery to high temperatures, or disassemble battery packs and cells.
●Do not apply pressure to the surface of the battery.
●Do not bend the battery.
●Do not use tools of any type to pry on or against the battery.
●If a battery gets stuck in a device as a result of swelling, do not try to free it as puncturing, bending, or crushing a battery 
can be dangerous.
●Do not attempt to reassemble a damaged or swollen battery into a laptop.
●Swollen batteries that are covered under warranty should be returned to Dell in an approved shipping container (provided 
by Dell)—this is to comply with transportation regulations. Swollen batteries that are not covered under warranty should be 
disposed of at an approved recycling center. Contact Dell product support at https://www.dell.com/support  for assistance 
and further instructions.
●Using a non-Dell or incompatible battery may increase the risk of fire or explosion. Replace the battery only with a 
compatible battery purchased from Dell that is designed to work with your Dell computer. Do not use a battery from other 
computers with your computer. Always purchase genuine batteries from https://www.dell.com  or otherwise directly from 
Dell.
Rechargeable Li-ion batteries can swell for various reasons such as age, number of charge cycles, or exposure to high heat. For 
more information about how to improve the performance and lifespan of the laptop battery and to minimize the possibility of 
occurrence of the issue, search Dell Laptop Battery in the Knowledge Base Resource at www.dell.com/support .
Locate the Service Tag or Express Service Code of 
your Dell computer
Your Dell computer is uniquely identified by a Service Tag or Express Service Code. To view relevant support resources for your 
Dell computer, we recommend entering the Service Tag or Express Service Code at www.dell.com/support .
For more information on how to find the Service Tag for your computer, see Locate the Service Tag for your Dell Laptop .9
128 Troubleshooting

Dell SupportAssist Pre-boot System Performance 
Check diagnostics
About this task
SupportAssist diagnostics (also known as system diagnostics) performs a complete check of your hardware. The Dell 
SupportAssist Pre-boot System Performance Check diagnostics is embedded with the BIOS and is launched by the BIOS 
internally. The embedded system diagnostics provides a set of options for particular devices or device groups allowing you to:
●Run tests automatically or in an interactive mode
●Repeat tests
●Display or save test results
●Run thorough tests to introduce additional test options to provide extra information about the failed device(s)
●View status messages that inform you if tests are completed successfully
●View error messages that inform you of problems encountered during testing
NOTE:  Some tests for specific devices require user interaction. Always ensure that you are present at the computer 
terminal when the diagnostic tests are performed.
For more information, see Resolve Hardware Issues With Integrated and Online Diagnostics (SupportAssist ePSA, ePSA, or PSA 
Error Codes) .
Running the SupportAssist Pre-Boot System Performance Check
Steps
1.Turn on your computer.
2.As the computer boots, press the F12 key as the Dell logo appears.
3.On the boot menu screen, select the Diagnostics  option.
4.Click the arrow at the bottom left corner.
Diagnostics front page is displayed.
5.Click the arrow in the lower-right corner to go to the page listing.
The items that are detected are listed.
6.To run a diagnostic test on a specific device, press Esc and click Yes to stop the diagnostic test.
7.Select the device from the left pane and click Run Tests .
8.If there are any issues, error codes are displayed.
Note the error code and validation number and contact Dell.
Built-in self-test (BIST)
M-BIST
M-BIST (Built In Self-Test) is the system board built-in self-test diagnostics tool that improves the diagnostics accuracy of 
system board Embedded Controller (EC) failures.
NOTE:  M-BIST can be manually initiated before Power On Self-Test (POST).
How to run M-BIST
NOTE:  M-BIST must be initiated on the computer from a power-off state that is either connected to AC power or with a 
battery only.
1.Press and hold both the M key on the keyboard and the power button  to initiate M-BIST.
2.The battery indicator LED may exhibit two states:
a.OFF: No fault was detected with the system board.
Troubleshooting 129

b.AMBER: Amber indicates a problem with the system board.
3.If there is a failure with the system board, the battery status LED flashes one of the following error codes for 30 seconds:
Table 44. LED error codes 
Blinking Pattern Possible Problem
Amber White
2 1 CPU Failure
2 8 LCD Power Rail Failure
1 1 TPM Detection Failure
2 4 Memory/RAM failure
4.If there is no failure with the system board, the LCD cycles through the solid color screens that are described in the 
LCD-BIST section for 30 seconds and then turn off.
LCD Power rail test (L-BIST)
L-BIST is an enhancement to the single LED error code diagnostics and is automatically initiated during POST. L-BIST will check 
the LCD power rail. If there is no power being supplied to the LCD (that is if the L-BIST circuit fails), the battery status LED 
flashes either an error code [2,8] or an error code [2,7].
NOTE:  If L-BIST fails, LCD-BIST cannot function as no power will be supplied to the LCD.
How to invoke the L-BIST Test:
1.Press the power button to start the computer.
2.If the computer does not start up normally, look at the battery status LED:
●If the battery status LED flashes an error code [2,7], the display cable may not be connected properly.
●If the battery status LED flashes an error code [2,8], there is a failure on the LCD power rail of the system board, hence 
there is no power that is supplied to the LCD.
3.For cases, when a [2,7] error code is shown, check to see if the display cable is properly connected.
4.For cases when a [2,8] error code is shown, replace the system board.
LCD Built-in Self-Test (BIST)
Dell laptops have a built-in diagnostic tool that helps you determine if the screen abnormality you are experiencing is an inherent 
problem with the LCD (screen) of the Dell laptop or with the video card (GPU) and computer settings.
When you notice screen abnormalities like flickering, distortion, clarity issues, fuzzy or blurry image, horizontal or vertical lines, 
color fade and so on, it is always a good practice to isolate the LCD (screen) by running the Built-In Self-Test (BIST).
How to invoke the LCD BIST Test
1.Power off the Dell laptop.
2.Disconnect any peripherals that are connected to the laptop. Connect only the AC adapter (charger) to the laptop.
3.Ensure that the LCD (screen) is clean (no dust particles on the surface of the screen).
4.Press and hold the D key and Power on  the laptop to enter LCD built-in self-test (BIST) mode. Continue to hold the D key 
until the computer boots up.
5.The screen displays solid colors and change colors on the entire screen to white, black, red, green, and blue twice.
6.Then it displays the colors white, black, and red.
7.Carefully inspect the screen for abnormalities (any lines, fuzzy color, or distortion on the screen).
8.At the end of the last solid color (red), the computer shuts down.
NOTE:  Dell SupportAssist Preboot diagnostics upon launch initiates an LCD BIST first, expecting a user intervention to 
confirm functionality of the LCD.
130 Troubleshooting

System-diagnostic lights
Power and battery-status light
The power and battery status light indicates the power and battery status of the computer. These are the power states:
Solid white:  Power adapter is connected and the battery has more than 5% charge.
Amber:  Computer is running on battery and the battery has less than 5% charge.
Off:
●Power adapter is connected, and the battery is fully charged.
●Computer is running on battery, and the battery has more than 5% charge.
●Computer is in sleep state, hibernation, or turned off.
The power and battery-status light may blink amber or white according to pre-defined "beep codes" indicating various failures.
For example, the power and battery-status light blinks amber two times followed by a pause, and then blinks white three times 
followed by a pause. This 2,3 pattern continues until the computer is turned off, indicating no memory or RAM is detected.
The following table shows different power and battery-status light patterns and associated problems.
NOTE:  The following diagnostic light codes and recommended solutions are intended for Dell service technicians to 
troubleshoot problems. You should only perform troubleshooting and repairs as authorized or directed by the Dell technical 
assistance team. Damage due to servicing that is not authorized by Dell is not covered by your warranty.
Table 45. Diagnostic-light LED codes 
Blinking pattern
Problem description Suggested resolution Amber White
2 1 CPU failure Replace the system board.
2 2 System board failure (included 
BIOS corruption or ROM 
error)●Flash latest BIOS version.
● If problem persists, 
replace the system board.
2 3 No memory/RAM detected ●Confirm that the memory 
module is installed 
properly.
● If problem persists, 
replace the memory 
module.
2 4 Memory/RAM failure Replace the memory module.
2 5 Invalid memory installed Replace the memory module.
2 6 System board/chipset error Replace the system board.
2 7 LCD failure Replace the LCD module.
2 8 LCD power rail failure Replace the LCD module.
3 1 RTC power failure Replace the battery.
3 2 PCI or video card/chip failure Replace the system board.
3 3 BIOS Recovery image not 
found●Flash latest BIOS version.
● If problem persists, 
replace the system board.
3 4 BIOS Recovery image found 
but invalid●Flash latest BIOS version.
Troubleshooting 131

Table 45. Diagnostic-light LED codes (continued)
Blinking pattern
Problem description Suggested resolution Amber White
● If problem persists, 
replace the system board.
3 5 Power rail failure ●Flash latest BIOS version.
● If problem persists, 
replace the system board.
3 6 SBIOS flash corruption ●Flash latest BIOS version.
● If problem persists, 
replace the system board.
3 7 ME error ●Flash latest BIOS version.
● If problem persists, 
replace the system board.
NOTE:  Blinking 3-3-3 LEDs on Lock LED (Caps-Lock or Nums-Lock), Power button LED (without Fingerprint reader), and 
Diagnostic LED indicates failure to provide input during LCD panel test on Dell SupportAssist Pre-boot System Performance 
Check diagnostics.
Camera status light:  Indicates whether the camera is in use.
●Solid white—Camera is in use.
●Off—Camera is not in use.
Caps Lock status light:  Indicates whether Caps Lock is enabled or disabled.
●Solid white — Caps Lock enabled.
●Off — Caps Lock disabled.
Recovering the operating system
When your computer is unable to boot to the operating system even after repeated attempts, it automatically starts Dell 
SupportAssist OS Recovery.
Dell SupportAssist OS Recovery is a stand-alone tool that is preinstalled in all Dell computers that are installed with the Windows 
operating system. It consists of tools to diagnose and troubleshoot issues that may occur before your computer boots to the 
operating system. It enables you to diagnose hardware issues, repair your computer, back up your files, or restore your computer 
to its factory state.
You can also download it from the Dell Support website to troubleshoot and fix your computer when it fails to boot into their 
primary operating system due to software or hardware failures.
For more information about the Dell SupportAssist OS Recovery, see Dell SupportAssist OS Recovery User's Guide  at 
www.dell.com/serviceabilitytools . Click SupportAssist  and then, click SupportAssist OS Recovery .
Real-Time Clock (RTC Reset)
The Real Time Clock (RTC) reset function allows you or the service technician to recover Dell computers from No POST/No 
Power/No Boot situations. The legacy jumper enabled RTC reset has been retired on these models.
Start the RTC reset with the computer powered off and connected to AC power. Press and hold the power button for twenty 
(20) seconds . The computer RTC Reset occurs after you release the power button.
132 Troubleshooting

Backup media and recovery options
It is recommended to create a recovery drive to troubleshoot and fix problems that may occur with Windows. Dell proposes 
multiple options for recovering the Windows operating system on your Dell computer. For more information, see Dell Windows 
Backup Media and Recovery Options .
Wi-Fi power cycle
About this task
If your computer is unable to access the Internet due to Wi-Fi connectivity issues a Wi-Fi power cycle procedure may be 
performed. The following procedure provides the instructions on how to conduct a Wi-Fi power cycle:
NOTE:  Some Internet Service Providers (ISPs) provide a modem or router combo device.
Steps
1.Turn off your computer.
2.Turn off the modem.
3.Turn off the wireless router.
4.Wait for 30 seconds.
5.Turn on the wireless router.
6.Turn on the modem.
7.Turn on your computer.
Drain residual flea power (perform hard reset)
About this task
Flea power is the residual static electricity that remains in the computer even after it has been powered off and the battery is 
removed.
For your safety, and to protect the sensitive electronic components in your computer, you are requested to drain residual flea 
power before removing or replacing any components in your computer.
Draining residual flea power, also known as a performing a "hard reset", is also a common troubleshooting step if your computer 
does not turn on or boot into the operating system.
Procedure to drain residual flea power (perform a hard reset)
Steps
1.Turn off your computer.
2.Disconnect the power adapter from your computer.
3.Remove the base cover.
4.Remove the battery.
5.Press and hold the power button for 20 seconds to drain the flea power.
6.Install the battery.
7.Install the base cover.
8.Connect the power adapter to your computer.
9.Turn on your computer.
NOTE:  For more information about performing a hard reset, search in the Knowledge Base Resource at www.dell.com/
support .
Troubleshooting 133

Getting help and contacting Dell
Self-help resources
You can get information and help on Dell products and services using these self-help resources:
Table 46. Self-help resources 
Self-help resources Resource location
Information about Dell products and services www.dell.com
My Dell app
Tips
Contact Support In Windows search, type Contact Support , and press 
Enter.
Online help for operating system www.dell.com/support/windows
www.dell.com/support/linux
Access top solutions, diagnostics, drivers and downloads, and 
learn more about your computer through videos, manuals, and 
documents.Your Dell computer is uniquely identified by a Service Tag or 
Express Service Code. To view relevant support resources for 
your Dell computer, enter the Service Tag or Express Service 
Code at www.dell.com/support .
For more information about how to find the Service Tag for 
your computer, see Locate the Service Tag on your computer .
Dell knowledge base articles 1.Go to www.dell.com/support .
2.On the menu bar at the top of the Support page, select 
Support  > Knowledge Base .
3.In the Search field on the Knowledge Base page, type the 
keyword, topic, or model number, and then click or tap the 
search icon to view the related articles.
Contacting Dell
To contact Dell for sales, technical support, or customer service issues, see www.dell.com/contactdell .
NOTE:  Availability varies by country/region and product, and some services may not be available in your country/region.
NOTE:  If you do not have an active Internet connection, you can find contact information about your purchase invoice, 
packing slip, bill, or Dell product catalog.10
134 Getting help and contacting Dell